# Water Quality Statistics

## Overview

We developed a set of inferential statistical models to estimate chemical concentrations in stormwater runoff across the Puget Sound watershed. Models were developed using spatial and temporal landscape predictors for a subset of constituents of concern (COCs). These models represent an advancement over current practices, which rely on categorical land use designations (e.g., high vs. low density residential, commercial, industrial) to approximate chemical concentrations in stormwater. While this method is convenient, it has some limitations. In particular, categorical land use designations vary by jurisdiction, limiting applicability of results between jurisdictions over a landscape. A qualitative comparison of existing land cover data sources and self-reported land use types revealed little agreement among jurisdictions’ definitions of land use categories. We therefore hypothesized that land use designation alone would be a poor predictor of chemical concentration in unmonitored basins.

Therefore, our objectives were to:

- Find a stronger, more data-driven method of predicting chemical concentration compared to the current method of using categorical land use designations to assign concentrations. Here, we investigate the potential for using continuous, measurable landscape spatial predictors (also referred to as spatial predictors) that can be objectively measured across large regions, thereby increasing the applicability of model results across jurisdictional boundaries.

- Determine whether a landscape spatial predictor approach produces stronger predictive power than either a null model or the current categorical land use model.

In order to achieve these objectives, we developed null, categorical land use, and landscape spatial predictor models to estimate concentrations for constituents of concern in Puget Sound stormwater. We define these models in the following way (details provided later):

- Null Model: COC concentration is equal to median COC concentration over all sampling dates and locations.

- Categorical Land Use Model: COC concentration is modeled as a linear mixed effects model, utilizing land use category (high density residential [HDR], low density residential [LDR], commercial [COM], industrial [IND]), precipitation (rainfall and/or antecedent dry days) and season (for COCs where a seasonal trend appeared in the data).

- Spatial Predictor Model: COC concentration is modeled as a linear mixed effects model, utilizing up to three landscape spatial predictors, precipitation (rainfall and/or antecedent dry days) and season (for COCs where a seasonal trend appeared in the data).

Landscape spatial predictors were drawn from various spatial data layers (e.g., traffic, forest cover, impervious surface cover), spatio-temporal layers (e.g., rainfall and seasonality), as well as land use (commercial, industrial, residential). These models were then compared against the null and categorical land use models for model fit and predictive ability. The best fit models were translated to the Stormwater Heatmap online communication tool using colors to show predicted pollutant concentration. For all COCs analyzed, the spatial predictor model out-performed the null model and the categorical land use model.

In addition to COC concentration estimates, the Stormwater Heatmap also provides stormwater managers with an understanding of the underlying variability in estimated COC concentrations presented on the Stormwater Heatmap. We achieved this on a sub-basin scale by calculating Bayesian 80% confidence intervals (known as credible intervals) for each sub-basin. This information can be accessed in the Stormwater Heatmap by choosing a geospatial sub-basin dataset to aggregate results (HUC12: USGS Watershed Boundary dataset; Washington Department of Ecology Puget Sound Assessment Units; or NHDPlus high resolution watershed dataset), and clicking on a sub-basin to view a pop-up window indicating the 80% upper and lower credible intervals for that sub-basin.

## Data Sources

### Outfall Data

The primary source of observed stormwater data is the S8.D Municipal Stormwater Permit Outfall Data (referred to as the S8 Data in this document) provided by the Washington Department of Ecology (Hobbs et al., 2015). Special Condition S8.D of the 2007-2012 Phase I Municipal Stormwater Permit required permittees to collect and analyze data to evaluate chemical concentrations in stormwater discharged from catchments with the following self-reported land uses: High Density Residential (HDR), Low Density Residential (LDR), Commercial (COM) and Industrial (IND). Phase I Permittees measured water quality, discharge, and other parameters from stormwater outfalls during storm events. Stormwater data collected from a total of 14 catchments by six Phase I Permittees were utilized in this modeling study.

The S8 Data are available from the Washington Department of Ecology (hereafter referred to as Ecology) via an open-data api at: https://data.wa.gov/Natural-Resources-Environment/Municipal-Stormwater- Permit-Outfall-Data/d958-q2ci.

COCs analyzed in this study are:

- Total Copper

- Total Suspended Solids (TSS)

- Total Phosphorus

- Total Zinc

- Total Kjeldahl Nitrogen (TKN)

We extracted data for these COCs, and performed minimal data cleaning. We filtered out rejected data (values with a R or REJ flag), removed replicates, and removed one data point for TKN that was an outlier (an order of magnitude higher than the majority of the data). Figure 4.1 shows data before removal of the outlier.

Figure 4.1 All observations with outlier in place for Total Kjeldahl nitrogen (TKN)

### Censored (Non-Detect) Data

Nearly all COCs had non-detect (ND; left-censored) data present (Table 4.1). Ecology flagged non-detect data and provided the reporting limit for each non-detect value. The EPA allows substitution of (0.5 x detection limit) for censored values when less than 15% of samples are non-detect (USEPA, 2009). However, substitution can add a pattern to the data that is different than the pattern of the data itself, with the likelihood increasing as the proportion of non-detect data increases (Helsel, 2012). In the S8 Data, most COCs had only a small percentage of data that were non-detect (<1.2%); for these, non-detect values were substituted with one-half of the reporting limit, which is allowed by EPA guidelines, providing less than 15% of samples are non-detect (USEPA, 2009). TKN had a higher level of non-detect data (approximately 10%). We decided not to perform substitution of non-detect TKN samples to avoid introducing a pattern to the data that is different than the data itself (Helsel 2012). Instead, regression on order statistics (ROS) was employed to calculate statistics for TKN. (See the Total Kjeldahl Nitrogen section under the Results heading for details.)

Table 4.1 Percentage of data points that were non-detect (left-censored) for each COC. ND: Non-detect.

### Transformation of outfall data

Each COC was evaluated for its underlying distribution, to determine whether transformation would be beneficial prior to analysis. Quantile-quantile plots (QQ plots) using a normal distribution, gamma distribution, square-root transformed distribution, and log-transformed distribution were visually analyzed. For all COC’s, log-transformation was deemed an appropriate data transformation prior to model analysis (Fig. 4.2).

Figure 4.2 Quantile-quantile plots of COCs using a natural-log (ln) scale. Red line shows the QQ-line.

## Landscape Spatial Data

For this study, we did not rely on the permittee’s self-reported land use type. As described earlier, a qualitative comparison of existing land cover data sources and self-reported land use types revealed little agreement among permittee definitions of the four land use types (HDR, LDR, COM, IND). Furthermore, areas with mixed land use types (e.g.: 30% commercial, 40% high density residential) could only correspond to one land use type. Therefore, we compiled a suite of continuous datasets for use in fitting and comparing models that estimate chemical concentration. We divide the datasets into two groups: continuous land use data and landscape data.

### Continuous Land Use Data

In order to employ a consistent analysis across different monitored watersheds we extracted land use data from the Washington Department of Commerce: Puget Sound Mapping Project (https://www.commerce.wa.gov/serving-communities/growth-management/puget-sound-mapping-project). Land use classes (also listed in Table 4.2) include:

- Intensive urban (includes commercial areas, apartment buildings)

- Urban residential (includes the majority of urban and suburban single-family dwellings)

- Rural residential

- Industrial

In addition, we combined some land use classes to generate additional land use classes:

- Total residential (urban residential + rural residential)

- Intensive Urban + Industrial (the industrial land use category only had a few non-zero data points)

### Landscape Data

For each watershed contained in the S8 Data, potentially relevant landscape data and their source are provided in Table 4.2. Additionally, several data sets were derived from these sources, including:

- totRES and intURB_IND — each of these are a grouping of similar land use categories (urban + rural residential; intensive urban + industrial) from the Washington State Department of Commerce: Puget Sound Mapping Project. Combining categories allowed us to combine some less common land uses from the S8 dataset (rural residential; industrial) with similar, more common land uses (urban residential; intensive urban);

- roof_urbRES, roof_totRES and roof_intURB – derived from rooftop density (Robertson et al, in prep) applied to land use data (Washington State Department of Commerce: Puget Sound Mapping Project);

- greenery — sum of low vegetation/grass, medium vegetation/shrub, and trees (Robertson et al, in prep);

- greeneryBE — sum of low vegetation/grass, medium vegetation/shrub, trees, and bare earth (e.g., exposed rock, mud flats, unvegetated fields); data from Robertson et al, in prep;

- not_greenBE — greeneryBE subtracted from 1, to yield the proportion of land cover that is not greenery or bare earth; data from Robertson et al, in prep;

- CO2_transport – sum of road, rail and commercial marine vessel CO2 data from Gurney et al., 2019;

- CO2_almostTotal – sum of commercial, residential, road, nonroad, rail and commercial marine vessel CO2 data from Gurney et al., 2019;

- devAge - calculated based on proportion of land developed by year range, using Pesaresi et al., 2005 data in the following equation: devAge = 4*dev_pre_1975 + 3*dev_1975_1990 + 2*dev_1990_2000 + 1*dev_2000_2014. Values close to 0 indicate little to no development, while values close to 4 indicate high or old development.

Table 4.2 Sources of landscape spatial predictor data used to develop statistical models.

### Pre-processing of spatial data

In order to use the landscape spatial data at an appropriate scale across the study area, spatial predictors were stacked and then convolved with a 100-meter gaussian kernel. This resulted in “fuzzy” predictors that could apply across dataset boundaries. These values were then extracted for each monitored watershed.

### Transformation

For some spatial data, the presence of outliers indicated that transformation would be beneficial. Spatial data were scaled and centered, then plotted and visually assessed for outliers (Fig. 4.3).

Figure 4.3 Untransformed, standardized spatial data. Standardization was performed by subtracting the mean of the 14 catchments, and dividing by the standard deviation.

Square-root transformation was performed on spatial data sets with outliers that were higher than the rest of the predictor’s values, including: slope, popn, CO2_almostTotal, CO2_com, CO2_nonroad, CO2_res, CO2_road, and CO2_transport. The devAge spatial predictor had a low data outlier; devAge values were squared to address the low outlier (devAge^2).

Additionally, traffic was square-root transformed. We decided on a square-root transformation for traffic for several reasons. 1.) From highway runoff data, we observed that heavy metal concentrations do not increase uniformly as traffic increases; rather, the slope looks as if it should be steeper at first, then gradually less steep at higher traffic levels (see Fig. 4 in Kayhanian et al., 2003; see also Fig. 3-1 in CalTrans 2003). This indicates that a weaker transformation for the traffic predictor (e.g., square-root transformation) would be a good choice in conjunction with the stronger transformation (ln-transformation) that was needed for COCs (dependent variable). 2.) The catchments represented by the S8 Data have traffic values up to 20,000 AADT. However, the mapped Stormwater Heatmap domain has traffic levels up to 160,000. Square-root transformation of the traffic predictor leads to a more linear pattern in credible interval ranges as traffic levels increase, while the untransformed traffic predictor has a more exponential pattern in credible interval ranges at higher traffic levels. Considering patterns in the highway runoff data, we decided that the square-root transformation would be appropriate for traffic.

Figure 4.4 shows the spatial data following transformation. Prior to use in models, spatial data sets were scaled and centered using the mean and standard deviation from the 14 catchments in this study.

Figure 4.4 Spatial data following transformation and standardization.

### Precipitation Data

Daily rainfall data were obtained from the Daymet website operated by NASA (https://daymet.ornl.gov/; Thornton et al 2022). Daily data were obtained for years 2009 to 2013, and cumulative one-day, three-day, seven-day, 14-day, 21-day and 28-day antecedent precipitation was calculated for each sampling date. For example, for a sampling date of March 24, cumulative three-day precipitation would include precipitation occurring on March 22, 23 and 24. For each COC, ln-transformed COC concentrations were plotted against each precipitation measurement, and the plots were visually assessed to select the most appropriate time scale for precipitation. Cumulative 21-day precipitation was selected for copper and phosphorus, cumulative 14-day precipitation was selected for zinc and TKN, and cumulative 1-day precipitation was selected for TSS.

## Model Construction and Selection

The structure of the data, with multiple data points over time for each of the 14 catchments, indicated that mixed effects models would be appropriate. We considered both a frequentist statistical framework, as well as a Bayesian framework for running the mixed effects models, and ended up utilizing a combination of the two. The frequentist framework is significantly faster to perform, and was used to identify the best set of spatial predictors as fixed effects, as well as to identify the best variance structure (see Steps 1-7, below).

Once the best fit model was selected, we fit the same model in a Bayesian framework; this allowed us to compute credible intervals around estimated COC values. Frequentist statistical methods are not able to properly estimate variability around random effects, so Bayesian methods were needed for this step.

Within a frequentist statistical framework, selection of the best model for each COC was performed using the methodology of Zuur et al. (2009), as outlined in Steps 1-7 below. Statistical analyses and model fitting were performed in R (R Core Team, 2021). Following the frequentist analysis, we moved into a Bayesian framework for Steps 8-9, which included model examination and credible interval calculations.

### Step 1. Select strong potential spatial predictors

The initial step was to find a suitable set of potential spatial predictors for each of the COCs evaluated. Single-predictor linear models were constructed for the relationship between ln-transformed chemical concentration and each spatial predictor, in turn. Plots were visually assessed to determine candidate predictors for linear mixed effects models. We looked for slopes with a p-value of < 0.05, and for data points to fall roughly along the best fit model line.

### Step 2. Construct a beyond-optimal model, controlling for collinearity

The starting point of our model selection process was a “beyond-optimal” model, where the fixed component of the mixed effects model contained as many strong spatial predictors as possible, as well as weather-based spatio-temporal predictors with potential importance (cumulative precipitation, season, antecedant dry days):

ln(C)ijk=fixed.effectsijk+ϵijk

Where:

C = concentration of chemical

i = sample

j = monitored catchment (location)

k = reporting agency, and

ϵijk = unexplained error

To prevent using highly correlated spatial predictors together in the beyond-optimal model, we calculated the correlation coefficient for each pair of spatial predictors and eliminated select predictors to reduce correlation. We then constructed a generalized least squares (gls) model in R (nlme package, Pinheiro et al., 2021) using the beyond-optimal set of fixed effects and utilizing restricted maximum likelihood estimation (REML).

Throughout the model selection process, highly correlated spatial predictors (correlation coefficient ≥0.85) were not used together in any models.

### Step 3. Control for heterogeneity (heteroskedasticity)

Normalized residuals from the beyond-optimal gls model were plotted against the model’s fitted values and examined for heteroskedasticity. We also plotted residuals against reporting agency, year, season, land use, cumulative precipitation, antecedent dry days, and all potential spatial predictors, to look for patterns in residuals. If residual plots showed evidence of heteroskedasticity, we tried a series of gls models (fitted with REML) with different variance structures that utilized reporting agency, location, season, and precipitation as variance covariates. Variance structure for each COC was selected based on the lowest Akaike information criterion (AIC) and Bayesian information criterion (BIC) scores.

If no heteroskedasticity was present, residuals were assumed to be normally distributed, with mean 0 and standard deviation

σ

ϵijk∼N(0,σ)

Controlling for heteroskedasticity often involved allowing each location j or each reporting agency k to have a different standard deviation, such that:

sd(ϵijk)=σj;   ϵijk∼N(0,σj)

or

sd(ϵijk)=σk;   ϵijk∼N(0,σk)

### Step 4. Find the proper random effects structure

Next, we used the beyond-optimal model with the correct variance structure to find the proper random effects structure. In the case of our data set, we had six sampling agencies, each with 1 to 3 locations, and multiple samples collected over time at each location. The nested structure of our data, along with our interest in using the model to predict results in locations that haven’t been sampled (rather than to compare sampled locations and agencies to each other), led to the identification of reporting agency and location as potential nested random effects. In this situation, the random part of a nested mixed effects model has the form:

random.effectsijk=ak+aj|k+ϵijk

where ak is a random intercept that allows for variation between agencies, while aj|k represents variation between locations (j) from the same reporting agency (k). We assumed that ak and aj|k are normally distributed, with standard deviations given below:

ak∼N(0,σagency)

aj|k∼N(0,σlocation)

The beyond-optimal gls model was compared to a linear mixed effects (lme) nested random-intercept model (nlme package, Pinheiro et al., 2021) fitted with REML, where the intercept was allowed to change per agency, and per location nested within reporting agency. Because the gls model and the random-intercept model are nested models, they were compared using a likelihood ratio test to see if one model was significantly better than the other (p-value < 0.05). The best-fit model was then selected based on the lowest AIC value. For all COCs, the nested random-intercept model fit the data better than the gls model with no random effects.

Intuitively, a random intercept for location nested within reporting agency is suitable for the stormwater data. Each location was sampled multiple times over the 4-year data collection period, and we would expect data within a location to be more similar than data between two locations. Additionally, reporting agencies may have differed slightly in sample collection methodologies, including: timing of sample collection once a storm began, selection of laboratory for sample analysis, and any biases in selection of representative catchments. These differences could lead to variability in COC results by reporting agency, and can be neatly captured in the random component of the model.

### Step 5. Check for temporal and spatial correlation

The beyond-optimal model with the correct variance structure and random effects structure was then examined for evidence of temporal and spatial auto-correlation. Temporal auto-correlation plots were generated and visually assessed for indications of correlation between various sampling time lags; no evidence of temporal autocorrelation was found for any COC. Spatial auto-correlation was assessed through two methods: variograms, and spatially-explicit bubble plots of model residuals; no compelling indications of spatial autocorrelation was found for any COC.

All tests for temporal and spatial auto-correlation were repeated following selection of best-fit models.

### Step 6. Find the proper fixed effects structure

We used the strong potential spatial predictors from Step 1 to generate an exhaustive set of fixed-effects formulae with combinations of one, two and three spatial predictors. Predictors with high correlation coefficients (≥0.85) were not placed into formulae together.

A set of models was generated using the fixed effects formulae, along with the best-fit random effects and variance structures identified in steps 3 and 4. Linear mixed effects models were applied using the nlme package (Pinheiro et al, 2021), and fitting with maximum likelihood (ML) estimation. Models where the sign (+ or –) of the spatial predictor coefficients did not match the coefficient signs from the linear model from Step 1 were discarded. Models were then sorted according to AIC value, and the top 20 models were examined for fit to individual predictors. If weak spatial or spatio-temporal predictors (p-value > 0.05) were present in the model, these were eliminated, then plots of residuals versus fitted values, reporting agency, location, and spatial predictors were examined. One to three candidate models were selected for consideration as the best spatial predictor model. Selection priority was given to simpler models to prevent over-fitting. In situations where models with one or two spatial predictors performed slightly worse than the best models with three spatial predictors, the models with less predictors were selected.

Candidate models all had low AIC values, acceptable residual plots, and good fit to individual spatial predictors. The form of the candidate models was:

ln(C)ijk=fixed.effectsijk+ak+aj|k+ϵijk

Using knowledge of chemical contaminant sources and mobilization into stormwater, we selected the best spatial predictor model for each COC.

### Step 7. Final model comparison to categorical land use models

Three models were ultimately fitted with REML and compared for each COC. This allowed us to assess the suitability of models built with spatial predictors, and ultimately to determine whether COC approximations made with spatial landscape predictors offer an improvement over those made with categorical land use designations. The three models for each COC include:

- Null Model: COC concentration is equal to median COC concentration over all sampling dates and locations

- Categorical Land Use Model: COC concentration is modeled as a linear mixed effects model, utilizing land use category (HDR, LDR, COM, IND), precipitation (rainfall and/or antecedent dry days), and season (for COCs where a seasonal trend appeared in the data).

- Spatial Predictor Model: COC concentration is modeled as a linear mixed effects model, utilizing up to three spatial predictors, precipitation (rainfall and/or antecedent dry days), and season (for COCs where a seasonal trend appeared in the data).

We compared these three models using AIC, to determine which model had the best fit to data. For all COCs analyzed, the spatial predictor model out-performed the null model and the categorical land use model; thus, the spatial predictor model for each COC was pushed forward for Bayesian model analysis. In the case of total phosphorus, an even simpler model including only rainfall and seasonal components had a slight improvement over the spatial predictor model in the frequentist framework, so both models were pushed forward for Bayesian model analysis.

Bayesian models were expected to yield similar parameter values to the frequentist models. As previously mentioned, Bayesian analysis allows computation of confidence intervals (called credible intervals in Bayesian statistics) around estimated COC values when random effects are present. Frequentist mixed effects models are not capable of accurately estimating variability around random effects, so confidence intervals calculated using frequentist statistics underestimate the uncertainty around each COC estimate. As discussed in Step 6, given the structure of the S8 Data (repeat sampling over time at catchment locations), along with methodological differences in stormwater sampling that could introduce variability in COC values across agencies, we felt it critical to include random effects and their associated variability in the models and their precision estimates.

### Step 8. Bayesian model analysis

The best fit frequentist model structure from Step 7 (made up of the variance structure from Step 3, random effects structure from Step 4, and fixed effects structure from Step 6) was then applied to a Bayesian framework using the brms package in R (Buerkner, 2017). The brms package provides an R interface for performing Bayesian multi-level (mixed effects) regression models using the programming language Stan in the background (Stan Development Team, 2022).

The brms algorithm accepts a prior distribution for each parameter (we assumed an uninformative prior distribution for each spatial predictor: Normal distribution with mean = 0 and standard deviation = 10) and provides an output of 4000 sets of parameter estimates that collectively make up what is called the posterior. The parameter sets vary in their likelihood given the data, and the parameter set with the highest likelihood is used in the Stormwater Heatmap.

Models were examined for the following elements:

- model convergence and adequate sampling of the parameter space using posterior traces. These address the questions: did the model look at enough parameter values? Was it able to find a best fit model using the given predictors?

- appropriateness of the variance structure. For some COCs, the difference in AIC value between models using location vs. reporting agency as a variance covariate was small. In these cases, the best-fit fixed effects and random effects structures were applied to models with different variance structures. The resulting models were compared using the leave-one-out information criterion (LOOIC - similar to AIC, where lower values indicate better model fit) to identify the best variance structure.

- model diagnostics using leave-one-out cross-validation (LOOCV) to calculate the Pareto k shape parameter. The Pareto k shape parameter provides an assessment of reliability and convergence rate of the Bayesian model estimator: a Pareto k value is calculated for each data point, and influential outliers that may reduce the model’s reliability are identified by Pareto k values > 0.7. When we identified models with Pareto k values above 0.7, we fitted Bayesian models using an underlying Student-t distribution rather than the default Gaussian distribution, to see if this resolved the issue. The Student-t distribution has thicker tails, which can encompass some more extreme observations. Models utilizing the Student-t distribution include an extra parameter, ν, for describing the shape of the Student-t distribution; this extra parameter was estimated by the Bayesian model in cases where a model using the Student-t distribution provided the best fit.

- graphical posterior predictive checking. We generated density plots of observed data along with simulated values from the posterior predictive distribution, to ensure that model-generated COC estimates fit observed data. We also examined observed vs. estimated plots. Graphical posterior predictive checking addresses the questions: Did the model correctly approximate the posterior distribution? Knowing that models are useful fiction and always fail in some way, how does our model fail?

- calculate model precision using root mean square error (RMSE) and RMSE-observations standard deviation ratio (RSR). RMSE was calculated by squaring residuals, finding the average of the squared residuals, then taking the square-root of that average. RSR is a standardized version of RMSE, calculated as RSR=RMSE/standard deviation of observations.

- analysis of sensitivity and uncertainty. We examined three facets of sensitivity in the models:

  - Are there influential data points, and if so, are the model conclusions sensitive to those influential data points?

  - Does changing the variance structure result in a quantitative change to the parameters for spatial landscape predictors?

  - If our selected random effects structure (samples nested within outfall locations nested within agencies) is incorrect, does a simpler structure (samples nested within outfall locations) result in qualitative differences to the model outcome?

The Bayesian model with the best variance structure and appropriate underlying distribution (Gaussian or Student-t) was then used to generate the heatmap layer, and to calculate credible intervals.

### Step 9. Obtain Credible Intervals from Bayesian model

Using the Bayesian model from Step 8, we calculated 80% credible intervals (80% CIs; analogous to confidence intervals in frequentist statistics) for all watershed sub-basins in the heatmap region. The Stormwater Heatmap 80% CIs can be interpreted as follows: there is an 80% probability that the true (but unknown) mean COC concentration will lie within the credible interval, given the evidence provided by the observed data.

Spatial means by sub-basin (called sub-basin reductions) were calculated for spatial landscape predictors (e.g., traffic level, development age). Three sub-basin reductions with different boundaries were used: Puget Sound Assessment Units (PSAU), the USGS watershed boundary dataset (HUC12), and the National Hydrography Dataset Plus (NHDPlus).

Stormwater Heatmap credible intervals need to account for a variety of realistic precipitation regimes and seasonal variation (e.g., wet vs. dry season), because these factors are important predictors in most of the COC models. We generated a set of realistic precipitation/season regimes by randomly sampling 10% of the S8 Data (47 samples total) for combined rain and season conditions. Forty-two of the samples were randomly selected from non-summer months, and five samples were randomly selected from summer months (July, August, September); this approximated the split between wet and dry seasons in the S8 Data, where 11% of data points were collected during summer months. Precipitation/season regimes were combined with sub-basin reductions to generate a data frame where each sub-basin was paired with each of the 47 options for precipitation/season. We will refer to this data frame as SBR.PS (sub-basin reduction/ precipitation, season).

Credible intervals for each sub-basin reduction were calculated using the fitted function (brms package; Buerkner, 2017) applied to the SBR.PS data frame. The fitted function runs each entry in the SBR.PS data frame through all 4000 parameter combinations generated in the Bayesian model, producing 4000*47 estimates of COC concentration in each sub-basin. Credible interval estimates were grouped by sub-basin and summarized for upper 80% CI and lower 80% CI. In this way, we were able to integrate precipitation and season into the credible intervals.

Stormwater Heatmap 80% credible intervals are provided for new, unsampled locations, where the 80% interval corresponds to the mean COC concentration at the new location, assuming that the new location is sampled many times. The credible intervals account for two of the three sources of model uncertainty:

- uncertainty around the population-level intercept (β0) and around the fixed effects parameters associated with spatial landscape predictors β3 and β4; and

- uncertainty around the variance parameters of the groups: σagency and σlocation.

Residual (observation-level) variance, ϵijk, is not accounted for in 80% CIs.

### Step 10. Translating Bayesian models to the Stormwater Heatmap

Bayesian model equations are substantially simplified for the Stormwater Heatmap communication tool, which displays over-annual mean predicted COC concentrations. Spatio-temporal predictors (rain, season) are set to their mean value of 0, and the variance structure and nested random effects terms are also set to their mean values of 0. Thus, the generic COC equation:

ln(Cijk)=β0+β1*raini+β2*summeri+β3*predictor1ijk+β4*predictor2ijk+ak+aj|k+ϵijk

ak∼N(0,σagency)

aj|k∼N(0,σlocation)

ϵijk∼N(0,σk)

simplifies to:

ln(C)=β0+β3*predictor1+β4*predictor2

for the Stormwater Heatmap. See the section Translating Equations to the Stormwater Heatmap at the end of this chapter for more details.

## Results

Spatial predictor models provided the best fit to data for all COCs analyzed, when compared with null models and categorical land use models. Table 4.3 shows the spatial predictor model equations for over-annual mean COC concentrations, which are displayed in the pollutant layers of the Stormwater Heatmap. For most COCs, the full spatial predictor models that were fitted to all data points also include precipitation (rain) and season (summer/dry season vs. not summer/wet season).

Detailed model-fitting results for each COC are provided in the sections below.

Table 4.3 Equations used in the Stormwater Heatmap visual layers for over-annual mean concentrations of constituents of concern.

### Total Copper

Based on linear models of ln-transformed copper versus individual spatial predictors, the strong spatial predictors identified for copper include: totRES, intURB, intURB_IND, grass, greenery, impervious, nodev, sqrt_traffic, sqrt_popn, pm25_na, sqrt_CO2_com, sqrt_CO2_road, sqrt_CO2_nonroad, sqrt_CO2_almostTotal, sqrt_CO2_transport, devAge2, and roof_intURB_IND (Fig 4.5).

Figure 4.5 Strong spatial predictors for copper, showing linear model fit (blue line) for the relationship between ln-transformed copper concentration and each predictor in turn.

The precipitation predictor used for copper was 21-day cumulative precipitation. In addition, evidence of higher copper concentrations during summer led us to add summer as a categorical predictor to the copper model, where summer = 1 during July, August, September, and summer = 0 for all other months.

Residuals plotted against fitted values showed signs of slight heterogeneity (Fig 4.6, top left). Of the variance structures tested, the best fit allows residual variation to differ by location j.

sd(ϵijk)=σj

Figure 4.6 Normalized residuals from beyond-optimal model, with no variance structure (top left), with reporting agency as a variance covariate (top right), and with location as a variance covariate (bottom left). Location as the variance covariate provided the best model fit.

#### Frequentist Model Summary

The best model for copper is a random-intercept model, where the intercept of the linear model is allowed to vary by reporting agency (k), and by location within each reporting agency (j|k). No signs of temporal or spatial auto-correlation were detected in auto-correlation plots or variograms.

Two possible models emerged to capture the fixed effects:

ln(Cuijk)=β0+β1*raini+β2*summeri+β3*sqrt_CO2_roadijk+β4*devAge2ijk+ak+aj|k+ϵijk

ln(Cuijk)=β0+β1*raini+β2*summeri+β3*sqrt_trafficijk+β4*devAge2ijk+ak+aj|k+ϵijk

The first model’s AIC score was slightly lower than that of the second model (AIC=699.3 vs. 700.9, when models were fitted with ML); however, vehicle wear is more directly associated with copper in stormwater than are emissions. As a result, we selected the second model, with sqrt_traffic as a predictor. Figure 4.7 shows the model fit for each individual predictor, plotted against data points. Correlation between the two predictors was low (correlation = 0.3).

Figure 4.7 Single-predictor plots for copper, showing fit of the Spatial Predictor Model to each predictor in turn. Model fitting was performed using maximum likelihood (ML) estimation.

Comparisons between the Null Model, Categorical Land Use Model, and Spatial Predictor Model can be visualized through residuals (Fig. 4.8) and comparative metrics such as AIC. The residual plots clearly show good fit (residuals close to zero) for the Categorical Land Use Model and for the Spatial Predictor Model.

Figure 4.8 Copper model residuals for the Null Model, Categorical Land Use Model, and Spatial Predictor Model. Each bar represents one watershed, with colors representing reporting agencies. Model fitting was performed using maximum likelihood (ML) estimation.

Final frequentist model AIC values (fitted with REML) for the three models are: 1317.1 (Null Model), 721.9 (Categorical Land Use Model) and 718.0 (Spatial Predictor Model), with the Spatial Predictor Model showing an improvement over the Categorical Land Use Model. Final coefficient values for all three frequentist models are shown in Figure 4.9. The large variability in model coefficients for the Categorical Land Use Model (shown in blue) indicates lower predictive power of this model. As a result, the Spatial Predictor model outperforms the currently used Categorical Land Use-type model both in terms of AIC, as well as in its potential for generating tighter precision intervals around estimates of copper concentration. We selected this improved method of estimating COC loading for inclusion in the Stormwater Heatmap’s data visualizations.

Figure 4.9 Frequentist model coefficients for the Null Model (green), Categorical Land Use Model (blue), and Spatial Predictor Model (red). These coefficient values for linear mixed effects models are based on fitting with restricted maximum likelihood (REML) estimation, and may differ slightly from those fitted using maximum likelihood (ML) estimation.

#### Bayesian Model Summary

The Bayesian model for total copper follows a Student-t distribution with the form:

ln(Cuijk)=β0+β1*raini+β2*summeri+β3*sqrt_trafficijk+β4*devAge2ijk+ak+aj|k+ϵijk

where:

β0=2.33;   β1=−0.18;   β2=0.37;   β3=0.43;   β4=0.46

ak∼N(0,σagency);   σagency=0.21

aj|k∼N(0,σlocation);   σlocation=0.45

ϵijk∼t7.8(0,σj);   ln(σj)=η+uj;   σj=eη*euj;   η=−0.91;

uj∼N(0,σ0);   σ0=0.21

Rain is 21-day cumulative precipitation, standardized; summer is a factor with value = 1 for July, August, September, and value = 0 for all other months. Both spatial predictors (traffic and development age) were first transformed then standardized prior to use. Subscripts indicate sample (i), location (j) and reporting agency (k).

Random effects ak (agency-specific contribution to the intercept) and aj|k (location-specific contribution to the intercept) follow a normal (Gaussian) distribution, with mean equal to 0 and standard deviation equal to σagency for agency-specific contribution to the intercept, and σlocation for location-specific contribution to the intercept. The residual variance ϵijk represents remaining variation in the data that cannot be explained by the model variables. For the copper model, residual variance follows a Student-t distribution, which is defined by three parameters: shape (ν=7.8) , mean (μ=0) and scale (σ=σj). By imposing a variance structure with location (j) as the variance covariate, scale (σj) becomes a function of parameters η and uj, where eη represents population-level variability, and euj represents deviation of location j from the population-level variability. Finally, uj is normally distributed, with mean equal to 0 and standard deviation equal to σ0.

Density plots for the 4000 sets of parameter estimates generated by the Bayesian model are provided in the left half of Figs. 4.10 and 4.11. The peaks of a density plot show where the most likely values (along the x-axis) of a given parameter are concentrated, similar to a histogram. They also indicate how much variability exists around parameter estimates, shown by the shape of the curve (low and broad equals lots of variability; tall and narrow equals little variability). The y-axis on density plots represents the probability density function for the kernel density estimation, and can be disregarded for our purposes. Traces (right half of Figs. 4.10 and 4.11) indicate how well the parameter space was sampled in the four chains generated by the NUTS sampler (no U-turn sampling - an extension of Hamilton Monte Carlo) used when fitting brms models. Each chain generated 1000 sets of parameter estimates, for a total of 4000 sets of parameter estimates. For copper, the traces indicate that the sampling algorithm did not get stuck in one region of the parameter space, and that the parameter space was well sampled.

Figure 4.10 Posterior densities (left) and traces (right) for parameters associated with fixed effects, obtained from the best-fit Bayesian model for total copper. In the plots, b_intercept corresponds to β0, b_rain corresponds to β1, b_summer1 corresponds to β2, b_sqrt_traffic corresponds to β3, and b_devAge2 corresponds to β4.

Figure 4.11 Posterior densities (left) and traces (right) for parameters associated with random effects, obtained from the best-fit Bayesian model for total copper. Here, b_sigma_intercept represents η from the random effects equations, sd_agency__Intercept represents σagency, sd_agency:location__Intercept represents σlocation, sd_location__sigma_Intercept represents σ0, and nu is the Greek letter ν, representing shape of the Student-t distribution.

Fit of the Bayesian copper model to data can be assessed through RMSE (0.490) or RSR (0.506), or visualized in estimated versus observed plots (Figs. 4.12 and 4.13). We provide RSR as a measure of model precision, since RSR is a standardized form of RMSE. Singh et al. (2004) recommends that an RSR ≤ 0.50 indicates a well suited model, while Moriasi et al. (2007) elaborates on this recommendation, suggesting that an RSR ≤ 0.50 indicates a very good model, an RSR ≤ 0.60 indicates a good model, an RSR ≤ 0.70 indicates a satisfactory model, and RSR > 0.7 indicates an unsatisfactory model. According to these guidelines, the copper model (RSR=0.506) is a good model.

In plots 4.12 and 4.13, the estimated ln(copper) values (vertical axis) are generated using the fitted function (brms package; Buerkner 2017), which applies the spatial predictors associated with each data point (location, rain, summer, sqrt_traffic, devAge2) to all 4000 parameter sets that make up the Bayesian posterior. Mean estimated values, along with 95% quantiles, are then plotted against observed ln(copper) data (horizontal axis). Ideally, the points should form a swath along the 1:1 diagonal dashed line. This 1:1 line shows where estimated values exactly match the observed data. For copper, the estimated vs. observed plots show slightly better model fit at higher copper concentrations (Fig. 4.12), evident because points in the upper right quadrant are closer to the 1:1 line than points in the lower left quadrant. Within-location variability in the upper right quadrant of Figure 4.12 is lower than for the lower left quadrant. Locations with higher copper concentrations also have medium to high values of sqrt_traffic and high values of devAge2 (Fig. 4.13, top panels). This could be coincidental, or could be the result of high temporal variability in copper-contributing sources (e.g., traffic) in low-traffic locations.

Summer and rain do not display a consistent effect on the Bayesian model’s fit to observed data (Fig. 4.13, bottom panels).

Figure 4.12 Estimated vs. observed plot for the Bayesian copper model, with point color corresponding to location. Individual points show how the observed value for ln(copper) (horizontal axis) compares to the mean estimated value from the Bayesian model (vertical axis). Thin gray vertical lines associated with each point provide the 95% quantiles for each estimate. Note that residual error is excluded from the estimates, resulting in narrower 95% quantiles. The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

Figure 4.13 Estimated vs. observed plots for the Bayesian copper model, with colors showing the magnitude of one spatial predictor (sqrt_traffic, devAge2, rain or summer) at each data point. Individual points show how the observed value for ln(copper) compares to the mean estimated value from the Bayesian model. Diagonal dashed lines show the 1:1 relationship, where estimated=observed.

Copper concentration estimates rely on two spatial predictors: sqrt_traffic and devAge2, along with two spatio-temporal predictors: precipitation and season. We can visualize the effect of one spatial landscape predictor at a time on expected copper concentrations (Fig. 4.14). In this figure, expected mean copper concentrations as a function of traffic or development age are presented as colored credible interval bands, with S8 Data—used in fitting the Bayesian model—plotted as black points atop the credible intervals. In order to generate Figure 4.14, we integrated over the range of values in the S8 Data for rainfall, season, and the non-displayed spatial predictor, to produce 50%, 80% and 95% credible interval bands. For example, the left half of Figure 4.14 shows expected mean copper concentrations as a function of traffic, given the observed range of values for development age, rainfall and season.

It is important to point out that Figure 4.14 is not showing the Stormwater Heatmap credible intervals by sub-basin. Stormwater Heatmap credible intervals pertain to a specific sub-basin with known levels of sqrt_traffic and devAge2, thus are expected to be tighter than the ones shown in Figure 4.14, which integrate over a range of values for the non-displayed spatial predictor.

Figure 4.14 credible intervals were calculated using the epred_draws function (tidybayes package; Kay, 2022) applied to a simulated dataset with all combinations of the following predictors:

- 20% of rain & summer combinations randomly sampled from the S8 Data, to provide representative scenarios of rain and summer/ not summer conditions;

- eight equally-spaced sqrt_traffic values between minimum and maximum sqrt_traffic levels observed in the S8 Data;

- eight equally-spaced devAge2 values between the minimum and maximum devAge2 levels observed in the S8 Data.

Following computation of credible intervals, spatial predictors were unstandardized and back-transformed to generate the figure.

As discussed in Step 9 of Model Construction and Selection, credible intervals account for uncertainty around fixed effect parameters and around the variance parameters of the groups, but do not account for residual variance. This allows stormwater managers to assess the following:

- What is the likely range for mean copper at a heretofore unsampled watershed with a particular level of traffic (or development age)?

This is in line with stormwater management best practices, which manage for the over-annual impact on water bodies rather than for extreme events.

Figure 4.14 Credible intervals for total copper as a function of raw traffic (Annual Average Daily Traffic; left plot) and development age (right plot). Bands of color represent 50%, 80% and 95% credible intervals, while black points represent S8 Data that were used in fitting the Bayesian model. Expected copper concentrations as a function of traffic (left plot) integrate over rainfall, season, and development age. Expected copper concentrations as a function of development age (right plot) integrate over rainfall, season and traffic.

The Bayesian copper model has a population-level intercept β0=2.33. As a random-intercept model, the location-specific intercept is allowed to vary by reporting agency (k), and by catchment location within each reporting agency (j|k), and sums to: β0+ak+aj|k. Density plots for the location-specific intercept parameters are shown in Figure 4.15, along with the mean and 95% credible intervals for the population-level intercept. Seven of the 14 catchments have location-specific intercepts that overlap well with the population-level intercept: the majority of their density plots are shaded blue (see e.g., TAC_COM). Four additional catchments have approximately half of their density plot overlapping with that of the population-level intercept (see e.g., TAC_IND). The remaining three catchments have location-specific intercepts that are quite different from that of the population (see TAC_HDR, SNO_COM, SEA_IND). The deviation between location-specific intercepts and the population-level intercept gives some insight into how much variability may exist in y-intercept values at unsampled locations. Variability around the population-level intercept and around group-specific random effects (reporting agency and location) is captured in the credible intervals shown in Figure 4.14, as well as in sub-basin credible intervals presented on the Stormwater Heatmap. Implementing more monitoring across diverse watershed types would be beneficial to providing more precise COC estimates and credible intervals.

The Stormwater Heatmap utilizes only the population-level intercept to estimate mean copper concentrations at unsampled locations; Figure 4.15 is provided as a visual for how much variability exists between locations, in terms of location-specific intercept.

Figure 4.15 Density plots for the 14 location-specific intercept parameters, along with the median (thick vertical line) and 95% credible intervals (dashed vertical lines) for the population-level intercept. For each location, the mean intercept value is indicated with a black dot, and credible intervals are shown with thick (50% CI) and thin (95% CI) black horizontal lines. Density plots for each location are shaded blue for values that fall within the 95% CI for the population-level intercept, and gray for values that fall outside of the 95% CI for population-level intercept. Note that the population-level 95% CI does not incorporate variability around reporting agency σagency, or around location within each reporting agency σlocation; it is solely the 95% CI for the population-level intercept (i.e.: 95% of the time, the population-level intercept will fall between the given CI values).

In summary, the Stormwater Heatmap utilizes the spatial predictors sqrt_traffic and devAge2 to estimate the over-annual mean copper concentration. The equation used for visualizing the total copper layer is:

ln(Cu)=2.33+0.43*sqrt_traffic+0.46*devAge2

See Applying Mixed Effects Equations to the Stormwater Heatmap at the end of this document for more information on how the Bayesian model is simplified to this equation.

### Total Suspended Solids

Based on linear models of ln-transformed TSS versus individual predictors, the strong predictors identified for TSS include: totRES, sqrt_traffic, sqrt_popn, sqrt_CO2_res, sqrt_CO2_com, sqrt_CO2_road, sqrt_CO2_almostTotal, sqrt_CO2_transport, and devAge2 (Fig 4.16).

Figure 4.16 Strong predictors for TSS, showing linear model fit (blue line) for the relationship between ln-transformed TSS concentration and each predictor in turn.

The precipitation predictor used for TSS was 1-day cumulative precipitation. There was no evidence of seasonal patterns to TSS in stormwater.

Residuals plotted against fitted values showed signs of slight heterogeneity (Fig 4.17, top left plot). Of the variance structures tested, the best fit allows residual variation to differ by location j.

sd(ϵijk)=σj

Figure 4.17 Normalized residuals from beyond-optimal model, with no variance structure (top left), with reporting agency as a variance covariate (top right) and with location as a variance covariate (bottom left). Location as the variance covariate provided the best model fit.

The best model for TSS is a random-intercept model, where the intercept of the linear model is allowed to vary by reporting agency (k), and by location within each reporting agency (j|k). No signs of temporal or spatial auto-correlation were detected in auto-correlation plots or variograms.

Two possible models emerged to capture the fixed effects:

ln(TSSijk)=β0+β1*raini+β2*sqrt_trafficijk+β3*devAge2ijk+ak+aj|k+ϵijk

ln(TSSijk)=β0+β1*raini+β2*sqrt_CO2_roadijk+β3*devAge2ijk+ak+aj|k+ϵijk

The AIC score for these models (fitted with ML) was 1240.2 and 1245.7, respectively. We selected the first model as the most suitable for TSS. Figure 4.18 shows the model fit for each individual predictor, plotted against data points. Correlation between the two predictors was low (correlation coefficient = 0.3).

Figure 4.18 Single-predictor plots for TSS, showing fit of the Spatial Predictor Model to each predictor. Model fitting was performed using maximum likelihood (ML) estimation.

Comparisons between the Null Model, Categorical Land Use Model, and Spatial Predictor Model can be visualized through residuals (Fig. 4.19) and comparative metrics such as AIC. The residual plots clearly show good fit (residuals close to zero) for the Categorical Land Use Model and for the Spatial Predictor Model.

Figure 4.19 TSS model residuals for the Null Model, Categorical Land Use Model, and Spatial Predictor Model. Each bar represents one watershed, with colors representing reporting agencies. Model fitting was performed using maximum likelihood (ML) estimation.

Final frequentist model AIC values (fitted with REML) for the three models are: 1431.7 (Null Model), 1258.3 (Categorical Land Use Model) and 1252.8 (Spatial Predictor Model), with the Spatial Predictor Model showing significant improvement over the Categorical Land Use Model. Final coefficient values for all three frequentist models are shown in Figure 4.20. The larger variability around Categorical Land Use Model coefficients (shown in blue) indicates lower predictive power compared to the Spatial Predictor Model, because confidence intervals around TSS estimates will be large. We selected the Spatial Predictor Model to estimate COC loading in the Stormwater Heatmap’s data visualizations.

Figure 4.20 Model coefficients for the Null Model (green), Categorical Land Use Model (blue), and Spatial Predictor Model (red). Final coefficient values for linear mixed effects models are based on fitting with restricted maximum likelihood (REML) estimation, and may differ slightly from those fitted using maximum likelihood (ML) estimation.

#### Bayesian Model Summary

The Bayesian model for TSS is:

ln(TSSijk)=10.29+0.16*raini+0.38*sqrt_trafficijk+0.32*devAge2ijk+ak+aj|k+ϵijk

where:

β0=10.29;   β1=0.16;   β2=0.38;   β3=0.32;

ak∼N(0,σagency);   σagency=0.28

aj|k∼N(0,σlocation);   σlocation=0.38

ϵijk∼N(0,σj);   ln(σj)=η+uj;   σj=eη*euj;   η=−0.08;

uj∼N(0,σ0);   σ0=0.30

Rain is 1-day cumulative precipitation, standardized. Both spatial predictors were first transformed then standardized prior to use. Subscripts indicate sample (i), location (j) and reporting agency (k).

Random effects ak (agency-specific contribution to the intercept) and aj|k (location-specific contribution to the intercept) follow a normal (Gaussian) distribution, with mean equal to 0 and standard deviation equal to σagency for agency-specific contribution to the intercept, and σlocation for location-specific contribution to the intercept. The residual variance ϵijk represents remaining variation in the data that cannot be explained by the model variables. For the TSS model, residual variance follows a normal distribution. By imposing a variance structure with location (j) as the variance covariate, scale (σj) becomes a function of parameters η and uj, where eη represents population-level variability, and euj represents deviation of location j from the population-level variability. Finally, uj is normally distributed, with mean equal to 0 and standard deviation equal to σ0.

Density plots for the 4000 sets of parameter estimates generated by the Bayesian model are provided in the left half of Figs. 4.21 and 4.22. The peaks of a density plot show where the most likely values (along the x-axis) of a given parameter are concentrated, similar to a histogram. They also indicate how much variability exists around parameter estimates, shown by the shape of the curve (low and broad equals lots of variability; tall and narrow equals little variability). The y-axis on density plots represents the probability density function for the kernel density estimation, and can be disregarded for our purposes. Traces (right half of Figs. 4.21 and 4.22) indicate how well the parameter space was sampled in the four chains generated by the NUTS sampler (no U-turn sampling - an extension of Hamilton Monte Carlo) used when fitting brms models. Each chain generated 1000 sets of parameter estimates, for a total of 4000 sets of parameter estimates. For TSS, the traces indicate that the sampling algorithm did not get stuck in one region of the parameter space, and that the parameter space was well sampled.

Figure 4.21 Posterior densities (left) and traces (right) for parameters associated with fixed effects, obtained from the best-fit Bayesian model for TSS. In the plots, b_intercept corresponds to β0, b_rain corresponds to β1, b_sqrt_traffic corresponds to β2, and b_devAge2 corresponds to β3.

Figure 4.22 Posterior densities (left) and traces (right) for parameters associated with random effects, obtained from the best-fit Bayesian model for TSS. Here, b_sigma_intercept represents η from the random effects equations, sd_agency__Intercept represents σagency, sd_agency:location__Intercept represents σlocation, and sd_location__sigma_Intercept represents σ0.

Fit of the Bayesian TSS model to data can be assessed through RMSE (0.961) or RSR (0.807), or visualized in estimated versus observed plots (Figs. 4.23 and 4.24). We provide RSR as a measure of model precision, since RSR is a standardized form of RMSE. Moriasi et al. (2007) suggest that an RSR ≤ 0.70 indicates a satisfactory model, while an RSR > 0.7 indicates an unsatisfactory model. According to these guidelines, the TSS model (RSR=0.81) is unsatisfactory.

In plots 4.23 and 4.24, the estimated ln(TSS) values (vertical axis) are generated using the fitted function (brms package; Buerkner 2017), which applies the spatial predictors associated with each data point (location, rain, sqrt_traffic, devAge2) to all 4000 parameter sets that make up the Bayesian posterior. Mean estimated values, along with 95% quantiles, are then plotted against observed ln(TSS) data (horizontal axis). Ideally, the points should form a swath along the 1:1 diagonal dashed line. This 1:1 line shows where estimated values exactly match the observed data.

For TSS, the estimated vs. observed plots show a broad swath of observed TSS values in locations with lower values for the spatial predictors traffic and devAge2 (see pastel colors in the top two plots of Fig. 24), and a slightly narrower swath of observed TSS values in locations with high values for traffic and devAge2 (see dark colors in the top two plots of Fig. 24). The broad distribution of points around the 1:1 line indicates that the model is not capturing all contributors to TSS. This is especially true for the bottom half of the estimated vs. observed plots, which represent low-traffic and/or less-developed locations (e.g., SNO_LDR, SNO_HDR, KIC_LDR, POT_COM, PIE_COM).

Cumulative one-day rainfall does not appear to affect the Bayesian model’s fit to observed data (Fig. 4.24, bottom panel).

Figure 4.23 Estimated vs. observed plot for the Bayesian TSS model, with point color corresponding to location. Individual points show how the observed value for ln(TSS) (horizontal axis) compares to the mean estimated value from the Bayesian model (vertical axis). Thin gray vertical lines associated with each point provide the 95% quantiles for each estimate. Note that residual error is excluded from the estimates, resulting in narrower 95% quantiles. The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

Figure 4.24 Estimated vs. observed plots for the Bayesian TSS model, with colors showing the magnitude of one spatial predictor (sqrt_traffic, devAge2 or rain) at each data point. Individual points show how the observed value for ln(TSS) compares to the mean estimated value from the Bayesian model. Diagonal dashed lines show the 1:1 relationship, where estimated=observed.

TSS concentration estimates rely on two spatial predictors: sqrt_traffic and devAge2, along with one spatio-temporal predictor: cumulative one-day precipitation. We can visualize the effect of one spatial landscape predictor at a time on expected TSS concentrations (Fig. 4.25). In this figure, expected mean TSS concentrations as a function of traffic or development age are presented as colored credible interval bands, with S8 Data—used in fitting the Bayesian model—plotted as black points atop the credible intervals. In order to generate Figure 4.25, we integrated over the range of values in the S8 Data for rainfall and the non-displayed spatial predictor, to produce 50%, 80% and 95% credible interval bands. For example, the left half of Figure 4.25 shows expected mean TSS concentrations as a function of traffic, given the observed range of values for development age and rainfall.

It is important to point out that Figure 4.25 is not showing the Stormwater Heatmap credible intervals by sub-basin. Stormwater Heatmap credible intervals pertain to a specific sub-basin with known levels of sqrt_traffic and devAge2, thus are expected to be tighter than the ones shown in Figure 4.25, which integrate over a range of values for the non-displayed spatial predictor.

Figure 4.25 credible intervals were calculated using the epred_draws function (tidybayes package; Kay, 2022) applied to a simulated dataset with all combinations of the following predictors:

- 20% of rain conditions randomly sampled from the S8 Data, to provide representative scenarios of rain conditions;

- eight equally-spaced sqrt_traffic values between minimum and maximum sqrt_traffic levels observed in the S8 Data;

- eight equally-spaced devAge2 values between the minimum and maximum devAge2 levels observed in the S8 Data.

Following computation of credible intervals, spatial predictors were unstandardized and back-transformed to generate the figure.

As discussed in Step 9 of Model Construction and Selection, credible intervals account for uncertainty around fixed effect parameters and around the variance parameters of the groups, but do not account for residual variance. This allows stormwater managers to assess the following:

- What is the likely range for mean TSS at a heretofore unsampled watershed with a particular level of traffic (or development age)?

This is in line with stormwater management best practices, which manage for the over-annual impact on water bodies rather than for extreme events.

Figure 4.25 Credible intervals for TSS as a function of raw traffic (Annual Average Daily Traffic; left plot) and development age (right plot). Bands of color represent 50%, 80% and 95% credible intervals, while black points represent S8 Data that were used in fitting the Bayesian model. Expected TSS concentrations as a function of traffic (left plot) integrate over rainfall and development age. Expected TSS concentrations as a function of development age (right plot) integrate over rainfall and traffic.

The Bayesian TSS model has a population-level intercept β0=10.29. As a random-intercept model, the location-specific intercept is allowed to vary by reporting agency (k), and by catchment location within each reporting agency (j|k), and sums to: β0+ak+aj|k. Density plots for the location-specific intercept parameters are shown in Figure 4.26, along with the mean and 95% credible intervals for the population-level intercept.

Twelve of the 14 catchments have location-specific intercepts that overlap well with the population-level intercept: the majority of their density plots are shaded blue (see e.g., TAC_HDR). The remaining two catchments have slightly less than half of their density plot overlapping with that of the population-level intercept (see e.g., SEA_IND). The deviation between location-specific intercepts and the population-level intercept gives some insight into how much variability may exist in y-intercept values at unsampled locations. In the case of TSS, the y-intercept values are relatively consistent regardless of location.

The Stormwater Heatmap utilizes only the population-level intercept to estimate mean TSS concentrations at unsampled locations; Figure 4.26 is provided as a visual for how much variability exists between locations, in terms of location-specific intercept.

Figure 4.26 Density plots for the 14 location-specific intercept parameters, along with the median (thick vertical line) and 95% credible intervals (dashed vertical lines) for the population-level intercept. For each location, the mean intercept value is indicated with a black dot, and credible intervals are shown with thick (50% CI) and thin (95% CI) black horizontal lines. Density plots for each location are shaded blue for values that fall within the 95% CI for the population-level intercept, and gray for values that fall outside of the 95% CI for population-level intercept. Note that the population-level 95% CI does not incorporate variability around reporting agency σagency, or around location within each reporting agency σlocation; it is solely the 95% CI for the population-level intercept (i.e.: 95% of the time, the population-level intercept will fall between the given CI values).

In summary, the Stormwater Heatmap utilizes the spatial predictors sqrt_traffic and devAge2 to estimate the over-annual mean TSS concentration. The equation used for visualizing the TSS layer is:

ln(TSS)=10.29+0.38*sqrt_traffic+0.32*devAge2

See Applying Mixed Effects Equations to the Stormwater Heatmap at the end of this document for more information on how the Bayesian model is simplified to this equation.

### Total Phosphorus

Based on linear models of ln-transformed total phosphorus (hereafter referred to only as phosphorus) versus individual predictors, the strong predictors identified for phosphorus include: urbRES, roofs, sqrt_traffic, sqrt_popn, pm25_na, sqrt_slope, sqrt_CO2_res, sqrt_CO2_road, sqrt_CO2_almostTotal, sqrt_CO2_transport, and devAge2 (Fig 4.27).

Figure 4.27 Strong predictors for phosphorus, showing linear model fit (blue line) for the relationship between ln-transformed phosphorus concentration and each predictor in turn.

The precipitation predictor used for phosphorus was 21-day cumulative precipitation. In addition, evidence of higher phosphorus concentrations during summer led us to add summer as a categorical predictor to the phosphorus model (where summer = 1 during July, August, September, and summer = 0 for all other months).

Residuals plotted against fitted values showed signs of slight heterogeneity (Fig 4.28, left plot). Of the variance structures tested, the best fit allows residual variation to differ by location j:

sd(ϵijk)=σj

Figure 4.28 Normalized residuals from beyond-optimal model, with no variance structure (top left), with reporting agency as a variance covariate (top right) and with location as a variance covariate (bottom left). Location as the variance covariate provided the best model fit.

The best model for phosphorus is a random-intercept model, where the intercept of the linear model is allowed to vary by reporting agency (k), and by location within each reporting agency (j|k). No signs of temporal or spatial auto-correlation were detected in auto-correlation plots or variograms.

Two possible models emerged to capture the fixed effects:

ln(Pijk)=β0+β1*raini+β2*summeri+β3*sqrt_CO2_roadijk+ak+aj|k+ϵijk

ln(Pijk)=β0+β1*raini+β2*summeri+ak+aj|k+ϵijk

The AIC score for the first model was slightly lower than that of the second model (AIC=813.6 vs. 814.7, when models were fitted with ML). In the first model, the predictor sqrt_CO2_road has a p-value of 0.08 when fitted with ML, indicating that it can be dropped from the model. However, dropping the single spatial predictor results in a model with no predictive potential for the Stormwater Heatmap. As a result, we selected the model that includes sqrt_CO2_road. Information for the rain + summer only model is also provided below.

Figure 4.29 shows the model fit for sqrt_CO2_road, plotted against data points.

Figure 4.29 Single-predictor plots for phosphorus, showing fit of the Spatial Predictor Model to the single predictor. Model fitting was performed using maximum likelihood (ML) estimation.

Comparisons between the Null Model, Categorical Land Use Model, and Spatial Predictor Model can be visualized through residuals (Fig. 4.30) and comparative metrics such as AIC. The residual plots clearly show good fit (residuals close to zero) for the Categorical Land Use Model, Spatial Predictor Model, and the Rain + Summer Only Model.

Figure 4.30 Phosphorus model residuals for the Null Model, Categorical Land Use Model, Spatial Predictor Model, and Rain + Summer Only Model (alternate model). Each bar represents one watershed, with colors representing agencies. Model fitting was performed using maximum likelihood (ML) estimation.

Final frequentist model AIC values (fitted with REML) for the four models are: 1157.2 (Null Model), 829.8 (Categorical Land Use Model), 825.1 (Spatial Predictor Model) and 824.0 (Rain + Summer Model). The Spatial Predictor Model shows an improvement over the Categorical Land Use Model, and the Rain + Summer Model has the lowest AIC, slightly improving over the Spatial Predictor Model. Final coefficient values for all four frequentist models are shown in Figure 4.31. We pushed both the Spatial Predictor Model and the Rain + Summer Model forward for Bayesian Model Selection. The close AIC values for these two particular models suggest that total phosphorus may be difficult to model using the S8 Data and available spatial landscape predictors. Phosphorus data from additional locations would be beneficial for model selection.

Figure 4.31 Frequentist model coefficients for the Null Model (green), Categorical Land Use Model (blue), Spatial Predictor Model (red), and Rain + Summer Model (purple). These coefficient values for linear mixed effects models are based on fitting with restricted maximum likelihood (REML) estimation, and may differ slightly from those fitted using maximum likelihood (ML) estimation.

#### Bayesian Model Summary

The Spatial Predictor model is slightly better than the Rain + Summer Model in the Bayesian modeling framework when compared with the LOOCV.

The Bayesian Spatial Predictor Model for total phosphorous follows a Student-t distribution with the form:

ln(Pijk)=β0+β1*raini+β2*summeri+β3*sqrt_CO2_roadijk+ak+aj|k+ϵijk

where:

β0=4.55;   β1=−0.15;   β2=0.51;   β3=0.30

ak∼N(0,σagency);   σagency=0.55

aj|k∼N(0,σlocation);   σlocation=0.51

ϵijk∼t13.4(0,σj);   ln(σj)=η+uj;   σj=eη*euj;   η=−0.64;

uj∼N(0,σ0);   σ0=0.30

Rain is 21-day cumulative precipitation, standardized; and summer is a factor with value = 1 for July, August, September, and value = 0 for all other months. The CO2_road predictor was first square-root transformed, then standardized prior to use. Subscripts indicate sample (i), location (j) and reporting agency (k).

Random effects ak (agency-specific contribution to the intercept) and aj|k (location-specific contribution to the intercept) follow a normal (Gaussian) distribution, with mean equal to 0 and standard deviation equal to σagency for agency-specific contribution to the intercept, and σlocation for location-specific contribution to the intercept. The residual variance ϵijk represents remaining variation in the data that cannot be explained by the model variables. For the total phosphorus model, residual variance follows a Student-t distribution, which is defined by three parameters: shape (ν=13.4) , mean (μ=0) and scale (σ=σj). By imposing a variance structure with location (j) as the variance covariate, scale (σj) becomes a function of parameters η and uj, where eη represents population-level variability, and euj represents deviation of location j from the population-level variability. Finally, uj is normally distributed, with mean equal to 0 and standard deviation equal to σ0.

Density plots for the 4000 sets of parameter estimates generated by the Bayesian model are provided in the left half of Figs. 4.32 and 4.33. The peaks of a density plot show where the most likely values (along the x-axis) of a given parameter are concentrated, similar to a histogram. They also indicate how much variability exists around parameter estimates, shown by the shape of the curve (low and broad equals lots of variability; tall and narrow equals little variability). The y-axis on density plots represents the probability density function for the kernel density estimation, and can be disregarded for our purposes. Traces (right half of Figs. 4.32 and 4.33) indicate how well the parameter space was sampled in the four chains generated by the NUTS sampler (no U-turn sampling - an extension of Hamilton Monte Carlo) used when fitting brms models. Each chain generated 1000 sets of parameter estimates, for a total of 4000 sets of parameter estimates. For total phosphorus, the traces indicate that the sampling algorithm did not get stuck in one region of the parameter space, and that the parameter space was well sampled.

Figure 4.32 Posterior densities (left) and traces (right) for parameters associated with fixed effects, obtained from the best-fit Bayesian model for total phosphorus. In the plots, b_intercept corresponds to β0, b_rain corresponds to β1, b_summer1 corresponds to β2, and b_sqrt_CO2_road corresponds to β3.

Figure 4.33 Posterior densities (left) and traces (right) for parameters associated with random effects, obtained from the best-fit Bayesian model for total phosphorus. Here, b_sigma_intercept represents η from the random effects equations, sd_agency__Intercept represents σagency, sd_agency:location__Intercept represents σlocation, sd_location__sigma_Intercept represents σ0, and nu is the Greek letter ν, representing shape of the Student-t distribution.

Fit of the Bayesian phosphorus model to data can be assessed through RMSE (0.607) or RSR (0.657), or visualized in estimated versus observed plots (Figs. 4.34 and 4.35). We provide RSR as a measure of model precision, since RSR is a standardized form of RMSE. Singh et al. (2004) recommends that an RSR ≤ 0.50 indicates a well suited model, while Moriasi et al. (2007) elaborates on this recommendation, suggesting that an RSR ≤ 0.50 indicates a very good model, an RSR ≤ 0.60 indicates a good model, an RSR ≤ 0.70 indicates a satisfactory model, and RSR > 0.7 indicates an unsatisfactory model. According to these guidelines, the phosphorus model (RSR=0.657) is a satisfactory model.

In plots 4.34 and 4.35, the estimated ln(phosphorus) values (vertical axis) are generated using the fitted function (brms package; Buerkner 2017), which applies the spatial predictors associated with each data point (location, rain, summer, sqrt_CO2_road) to all 4000 parameter sets that make up the Bayesian posterior. Mean estimated values, along with 95% quantiles, are then plotted against observed ln(phosphorus) data (horizontal axis). Ideally, the points should form a swath along the 1:1 diagonal dashed line. This 1:1 line shows where estimated values exactly match the observed data. For phosphorus, the estimated vs. observed plots show slightly better model fit at higher phosphorus concentrations (Fig. 4.34), evident because points in the upper right quadrant are closer to the 1:1 line than points in the lower left quadrant. Within-location variability in the upper right quadrant of Figure 4.34 (e.g., SEA_COM) is somewhat lower than for the lower left quadrant (e.g., POT_COM). The model seems to better estimate phosphorus concentrations in locations with high values of sqrt_CO2_road (Fig. 4.35, top panel).

Summer and rain do not display a consistent effect on the Bayesian model’s fit to observed data (Fig. 4.35, bottom panels).

Figure 4.34 Estimated vs. observed plot for the Bayesian total phosphorus model, with point color corresponding to location. Individual points show how the observed value for ln(phosphorus) (horizontal axis) compares to the mean estimated value from the Bayesian model (vertical axis). Thin gray vertical lines associated with each point provide the 95% quantiles for each estimate. Note that residual error is excluded from the estimates, resulting in narrower 95% quantiles. The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

Figure 4.35 Estimated vs. observed plots for the Bayesian total phosphorus model, with colors showing the magnitude of one spatial predictor (sqrt_CO2_road, rain or summer) at each data point. Individual points show how the observed value for ln(phosphorus) compares to the mean estimated value from the Bayesian model. Diagonal dashed lines show the 1:1 relationship, where estimated=observed.

Phosphorus concentration estimates rely on the spatial predictor sqrt_CO2_road, along with two spatio-temporal predictors: precipitation and season. Figure 4.36 visualizes the effect of sqrt_CO2_road on expected phosphorus concentrations as colored credible interval bands, with S8 Data—used in fitting the Bayesian model—plotted as black points atop the credible intervals. In order to generate Figure 4.36, we integrated over the range of values in the S8 Data for rainfall and season to produce 50%, 80% and 95% credible interval bands.

Because only a single spatial landscape predictor is utilized in the phosphorus model, the credible intervals shown in Figure 4.36 will be similar to those presented by sub-basin on the Stormwater Heatmap.

Figure 4.36 credible intervals were calculated using the epred_draws function (tidybayes package; Kay, 2022) applied to a simulated dataset with all combinations of the following predictors:

- 20% of rain & summer combinations randomly sampled from the S8 Data, to provide representative scenarios of rain and summer/ not summer conditions;

- eight equally-spaced sqrt_CO2_road values between minimum and maximum sqrt_CO2_road levels observed in the S8 Data.

Following computation of credible intervals, the spatial predictor sqrt_CO2_road was unstandardized and back-transformed to generate the figure.

As discussed in Step 9 of Model Construction and Selection, credible intervals account for uncertainty around fixed effect parameters and around the variance parameters of the groups, but do not account for residual variance. This allows stormwater managers to assess the following:

- What is the likely range for mean total phosphorus at a heretofore unsampled watershed with a particular level of onroad CO2?

This is in line with stormwater management best practices, which manage for the over-annual impact on water bodies rather than for extreme events.

Figure 4.36 Credible intervals for total phosphorus as a function of raw onroad CO2 emissions. Bands of color represent 50%, 80% and 95% credible intervals, while black points represent S8 Data that were used in fitting the Bayesian model. Expected phosphorus concentrations as a function of onroad CO2 emissions integrate over rainfall and season.

The Bayesian total phosphorus model has a population-level intercept β0=4.55. As a random-intercept model, the location-specific intercept is allowed to vary by reporting agency (k), and by catchment location within each reporting agency (j|k), and sums to: β0+ak+aj|k. Density plots for the location-specific intercept parameters are shown in Figure 4.37, along with the mean and 95% credible intervals for the population-level intercept.

Thirteen of the 14 catchments have location-specific intercepts that overlap well with the population-level intercept: the majority of their density plots are shaded blue (see e.g., TAC_IND). The remaining catchment (KIC_COM) has a location-specific intercept that is quite different from that of the population. The deviation between location-specific intercepts and the population-level intercept gives some insight into how much variability may exist in y-intercept values at unsampled locations. In the case of total phosphorus, the y-intercept values are relatively consistent regardless of location.

The Stormwater Heatmap utilizes only the population-level intercept to estimate mean phosphorus concentrations at unsampled locations; Figure 4.37 is provided as a visual for how much variability exists between locations, in terms of location-specific intercept.

Figure 4.37 Density plots for the 14 location-specific intercept parameters, along with the median (thick vertical line) and 95% credible intervals (dashed vertical lines) for the population-level intercept. For each location, the mean intercept value is indicated with a black dot, and credible intervals are shown with thick (50% CI) and thin (95% CI) black horizontal lines. Density plots for each location are shaded blue for values that fall within the 95% CI for the population-level intercept, and gray for values that fall outside of the 95% CI for population-level intercept. Note that the population-level 95% CI does not incorporate variability around reporting agency σagency, or around location within each reporting agency σlocation; it is solely the 95% CI for the population-level intercept (i.e.: 95% of the time, the population-level intercept will fall between the given CI values).

The Stormwater Heatmap utlizes the spatial predictor sqrt_CO2_road to estimate the over-annual mean total phosphorus concentration. The equation used for visualizing the total phosphorus layer is:

ln(P)=4.55+0.30*sqrt_CO2_road

See Applying Mixed Effects Equations to the Stormwater Heatmap at the end of this document for more information on how the Bayesian model is simplified to this equation.

### Total Zinc

Based on linear models of ln-transformed total zinc versus individual predictors, the strong predictors identified for total zinc include: totRES, intURB_IND, grass, greenery, not_greenBE, paved, impervious, trees, sqrt_traffic, pm25_na, sqrt_CO2_com, sqrt_CO2_nonroad, sqrt_CO2_almostTotal, sqrt_CO2_transport, roof_intURB_IND, and roof_totRES (Fig. 4.38).

Figure 4.38 Strong predictors for total zinc, showing linear model fit (blue line) for the relationship between ln-transformed total zinc concentration and each predictor in turn.

The precipitation predictor used for total zinc was 14-day cumulative precipitation. In addition, evidence of higher total zinc concentrations during summer led us to add summer as a categorical predictor to the total zinc model (where summer = 1 during July, August, and September, and summer= 0 for all other months).

Residuals plotted against fitted values showed a pattern, indicating a chance of slight heteroskedasticity (Fig. 4.39, top left). Of the variance structures tested, the best fit allows residual variation to differ by location j.

sd(ϵj)=σj

Figure 4.39 Normalized residuals from beyond-optimal model, with no variance structure (top left), with reporting agency as a variance covariate (top right), and with location as a variance covariate (bottom left). Location as the variance covariate provided the best model fit.

The best model for total zinc is a random-intercept model, where the intercept of the linear model is allowed to vary by reporting agency (k), as well as by location within each reporting agency (j|k). No signs of temporal or spatial auto-correlation were detected in auto-correlation plots or variograms.

We explored a variety of models for total zinc. Three possible models emerged to capture the fixed effects:

ln(Znijk)=β0+β1*raini+β2*summeri+β3*pavedijk+β4*sqrt_CO2_transportijk+β5*raini*pavedijk+ak+aj|k+ϵijk

ln(Znijk)=β0+β1*raini+β2*summeri+β3*pavedijk+β4*sqrt_trafficijk+β5*raini*pavedijk+ak+aj|k+ϵijk

ln(Znijk)=β0+β1*raini+β2*summeri+β3*greeneryijk+β4*sqrt_CO2_transportijk+β5*raini*greeneryijk+ak+aj|k+ϵijk

The AIC scores for these models were relatively close (AIC=701.0, 704.0 and 705.3, respectively, when fitted with ML). When the first two models were visualized on the Stormwater Heatmap, we noticed problems with the paved predictor: it has a linear relationship with zinc over the range of values in the S8 fitting data, but the relationship appears non-linear at higher values of paved. The third model (greenery and sqrt_CO2_transport) has some problems in non-vegetated open spaces (e.g., mudflats, rock formations in the mountains), where the model estimates high zinc concentrations even though there is no anthropogenic source of zinc.

Because the greenery predictor did well in the initial model fitting, we tried a new predictor that would eliminate the problem of estimating high concentrations in non-vegetated open spaces: greenery_bareEarth. This predictor is a sum of grass, medium-vegetation, trees and bare earth, and provided a good fit to the S8 data. For application to the Stormwater Heatmap, we used the inverse of this predictor: not_greenBE in conjunction with sqrt_CO2_transport. The AIC value for this model fit with ML is 705.4, and the equation is:

ln(Znijk)=β0+β1*raini+β2*summeri+β3*not_greenBEijk+β4*sqrt_CO2_transportijk+β5*raini*not_greenBEijk+ak+aj|k+ϵijk

Figure 4.40 shows the model fit for the not_greenBE and sqrt_CO2_transport spatial predictors, plotted against data points. Figure 4.41 shows the interaction between rain and not_greenBE.

Figure 4.40 Single-predictor plots for total zinc, showing fit of the Spatial Predictor Model to the not_greenBE and sqrt_CO2_transport predictors. Model fitting was performed using maximum likelihood (ML) estimation.

Figure 4.41 Plot showing the interaction between 14-day cumulative rainfall and not_greenBE, with watershed catchments that contain less greenery (higher not_greenBE value) shown in reds and oranges, and catchments with more greenery (lower not_greenBE value) shown in blues and purples. The steeper red and orange slopes indicate a stronger first-flush effect in highly developed/less green areas compared to areas with more greenery and less pavement.

Model fitting was performed using maximum likelihood (ML) estimation. Comparisons between the Null Model, Categorical Land Use Model, and Spatial Predictor Model can be visualized through residuals (Fig. 4.42) and comparative metrics such as AIC. The residual plots clearly show good fit (residuals close to zero) for the Categorical Land Use Model and for the Spatial Predictor Model.

Figure 4.42 Total zinc model residuals for the Null Model, Categorical Land Use Model, and Spatial Predictor Model. Each bar represents one watershed, with colors representing agencies. Model fitting was performed using maximum likelihood (ML) estimation.

Final frequentist model AIC values (fitted with REML) for the three models are: 1285.0 (Null Model), 723.4 (Categorical Land Use Model), and 726.9 (Spatial Predictor Model), with the Categorical Land Use Model showing a slightly better AIC value than the Spatial Predictor Model when fitted with REML (ML fitting has the opposite outcome). Final coefficient values for all three frequentist models are shown in Figure 4.43. The large variability in model coefficients for the Categorical Land Use Model (shown in blue) indicates low predictive power, because confidence intervals on predictions will be large. We selected the Spatial Predictor Model to estimate COC loading in the Stormwater Heatmap’s data visualizations for its lower AIC value, and the increase in predictive power.

Figure 4.43 Frequentist model coefficients for the Null Model (green), Categorical Land Use Model (blue), and Spatial Predictor Model (red). Final coefficient values for linear mixed effects models are based on fitting with REML, and may differ slightly from coefficient values based on fitting with ML.

#### Bayesian Model Summary

The Bayesian model for total zinc follows a Student-t distribution with the form:

ln(Znijk)=β0+β1*raini+β2*summeri+β3*sqrt_CO2_transportijk+β4*not_greenBEijk+β5*raini*not_greenBEijk+ak+aj|k+ϵijk

where:

β0=4.07;   β1=−0.16;   β2=0.34;   β3=0.45;   β4=0.42;   β5=−0.06

ak∼N(0,σagency);   σagency=0.24

aj|k∼N(0,σlocation);   σlocation=0.56

ϵijk∼t16.80(0,σj);   ln(σj)=η+uj;   σj=eη*euj;   η=−0.79;

uj∼N(0,σ0);   σ0=0.32

Rain is 14-day cumulative precipitation, standardized; and summer is a factor with value = 1 for July, August, September, and value = 0 for all other months. The spatial predictor CO2_transport was square-root transformed, and both predictors (sqrt_CO2_transport and not_greenBE) were standardized prior to use. Subscripts indicate sample (i), location (j) and reporting agency (k).

Random effects ak (agency-specific contribution to the intercept) and aj|k (location-specific contribution to the intercept) follow a normal (Gaussian) distribution, with mean equal to 0 and standard deviation equal to σagency for agency-specific contribution to the intercept, and σlocation for location-specific contribution to the intercept. The residual variance ϵijk represents remaining variation in the data that cannot be explained by the model variables. For the total zinc model, residual variance follows a Student-t distribution, which is defined by three parameters: shape (ν=16.80) , mean (μ=0) and scale (σ=σj). By imposing a variance structure with location (j) as the variance covariate, scale (σj) becomes a function of parameters η and uj, where eη represents population-level variability, and euj represents deviation of location j from the population-level variability. Finally, uj is normally distributed, with mean equal to 0 and standard deviation equal to σ0.

Density plots for the 4000 sets of parameter estimates generated by the Bayesian model are provided in the left half of Figs. 4.44 and 4.45. The peaks of a density plot show where the most likely values (along the x-axis) of a given parameter are concentrated, similar to a histogram. They also indicate how much variability exists around parameter estimates, shown by the shape of the curve (low and broad equals lots of variability; tall and narrow equals little variability). The y-axis on density plots represents the probability density function for the kernel density estimation, and can be disregarded for our purposes. Traces (right half of Figs. 4.44 and 4.45) indicate how well the parameter space was sampled in the four chains generated by the NUTS sampler (no U-turn sampling - an extension of Hamilton Monte Carlo) used when fitting brms models. Each chain generated 1000 sets of parameter estimates, for a total of 4000 sets of parameter estimates. For total zinc, the traces indicate that the sampling algorithm did not get stuck in one region of the parameter space, and that the parameter space was well sampled.

Figure 4.44 Posterior densities (left) and traces (right) for parameters associated with fixed effects, obtained from the best-fit Bayesian model for total zinc. In the plots, b_intercept corresponds to β0, b_rain corresponds to β1, b_summer1 corresponds to β2, b_sqrt_CO2_transport correspondes to β3, b_not_greenBE corresponds to β4, and b_rain:not_greenBE corresponds to β5.

Figure 4.45 Posterior densities (left) and traces (right) for parameters associated with random effects, obtained from the best-fit Bayesian model for total zinc. Here, b_sigma_intercept represents η from the random effects equations, sd_agency__Intercept represents σagency, sd_agency:location__Intercept represents σlocation, sd_location__sigma_Intercept represents σ0, and nu is the Greek letter ν, representing shape of the Student-t distribution.

Fit of the Bayesian total zinc model to data can be assessed through the precision metrics RMSE (0.51) or RSR (0.53), or visualized in estimated versus observed plots (Figs. 4.46 and 4.47). We provide RSR as a measure of model precision, since RSR is a standardized form of RMSE. Singh et al. (2004) recommends that an RSR ≤ 0.50 indicates a well suited model, while Moriasi et al. (2007) elaborates on this recommendation, suggesting that an RSR ≤ 0.50 indicates a very good model, an RSR ≤ 0.60 indicates a good model, an RSR ≤ 0.70 indicates a satisfactory model, and RSR > 0.7 indicates an unsatisfactory model. According to these guidelines, the total zinc model (RSR=0.53) is a good model.

In figures 4.46 and 4.47, the estimated ln(total zinc) values (vertical axis) are generated using the fitted function (brms package; Buerkner 2017), which applies the spatial predictors associated with each data point (location, rain, summer, sqrt_CO2_transport, not_greenBE) to all 4000 parameter sets that make up the Bayesian posterior. Mean estimated values, along with 95% quantiles, are then plotted against observed ln(total zinc) data (horizontal axis). Ideally, the points should form a swath along the 1:1 diagonal dashed line. This 1:1 line shows where estimated values exactly match the observed data. For total zinc, the estimated vs. observed plots show good model fit to data over all observed zinc concentrations. Several locations show higher within-location variability (e.g., KIC_LDR, KIC_HDR, SNO_COM in Fig. 4.47), but no pattern emerges indicating better model fit at higher or lower zinc concentrations.

Summer and rain do not display a consistent effect on the Bayesian model’s fit to observed data (Fig. 4.47, bottom panels).

Figure 4.46 Estimated vs. observed plot for the Bayesian total zinc model, with point color corresponding to location. Individual points show how the observed value for ln(total zinc) (horizontal axis) compares to the mean estimated value from the Bayesian model (vertical axis). Thin gray vertical lines associated with each point provide the 95% quantiles for each estimate. Note that residual error is excluded from the estimates, resulting in narrower 95% quantiles. The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

Figure 4.47 Estimated vs. observed plots for the Bayesian total zinc model, with colors showing the magnitude of one spatial predictor (not_greenBE, sqrt_CO2_transport, rain or summer) at each data point. Individual points show how the observed value for ln(total zinc) compares to the mean estimated value from the Bayesian model. Diagonal dashed lines show the 1:1 relationship, where estimated=observed.

Total zinc concentration estimates rely on two spatial predictors: sqrt_CO2_transport and not_greenBE, along with two spatio-temporal predictors: precipitation and season. We can visualize the effect of one spatial landscape predictor at a time on expected total zinc concentrations (Fig. 4.48). In this figure, expected mean zinc concentrations as a function of not_greenBE or CO2_transport are presented as colored credible interval bands, with S8 Data—used in fitting the Bayesian model—plotted as black points atop the credible intervals. In order to generate Figure 4.48, we integrated over the range of values in the S8 Data for rainfall, season, and the non-displayed spatial predictor, to produce 50%, 80% and 95% credible interval bands. For example, the left half of Figure 4.48 shows expected mean zinc concentrations as a function of not_greenBE, given the observed range of values for CO2_transport, rainfall and season.

It is important to point out that Figure 4.48 is not showing the Stormwater Heatmap credible intervals by sub-basin. Stormwater Heatmap credible intervals pertain to a specific sub-basin with known levels of not_greenBE and sqrt_CO2_transport, thus are expected to be tighter than the ones shown in Figure 4.48, which integrate over a range of values for the non-displayed spatial predictor.

Figure 4.48 credible intervals were calculated using the epred_draws function (tidybayes package; Kay, 2022) applied to a simulated dataset with all combinations of the following predictors:

- 20% of rain & summer combinations randomly sampled from the S8 Data, to provide representative scenarios of rain and summer/ not summer conditions;

- eight equally-spaced not_greenBE values between minimum and maximum not_greenBE observed in the S8 Data;

- eight equally-spaced sqrt_CO2_transport values between minimum and maximum sqrt_CO2_transport observed in the S8 Data.

Following computation of credible intervals, the spatial predictors were unstandardized and sqrt_CO2_transport was back-transformed to generate the figure.

As discussed in Step 9 of Model Construction and Selection, credible intervals account for uncertainty around fixed effect parameters and around the variance parameters of the groups, but do not account for residual variance. This allows stormwater managers to assess the following:

- What is the likely range for mean total zinc at a heretofore unsampled watershed with a particular level of not_greenBE (or CO2_transport)?

This is in line with stormwater management best practices, which manage for the over-annual impact on water bodies rather than for extreme events.

Figure 4.48 Credible intervals for total zinc as a function of not_greenBE (left plot; 0 indicates a watershed that is fully green or bare earth, and 1 indicates a fully impervious watershed) and CO2_transport (right plot). Bands of color represent 50%, 80% and 95% credible intervals, while black points represent S8 Data that were used in fitting the Bayesian model. Expected total zinc concentrations as a function of not_greenBE (left plot) integrate over rainfall, season, and CO2_transport. Expected zinc concentrations as a function of CO2_transport (right plot) integrate over rainfall, season and not_greenBE.

The Bayesian total zinc model has a population-level intercept β0=4.07. As a random-intercept model, the location-specific intercept is allowed to vary by reporting agency (k), and by catchment location within each reporting agency (j|k), and sums to: β0+ak+aj|k. Density plots for the location-specific intercept parameters are shown in Figure 4.50, along with the mean and 95% credible intervals for the population-level intercept. Seven of the 14 catchments have location-specific intercepts that overlap well with the population-level intercept: the majority of their density plots are shaded blue (see e.g., TAC_IND). Four additional catchments have approximately half of their density plot overlapping with that of the population-level intercept (e.g., SNO_LDR). The remaining three catchments (SNO_HDR, KIC_LDR, KIC_COM) have location-specific intercepts that are quite different from that of the population. Two of these (KIC_LDR, SNO_HDR) also show higher within-location variability in zinc concentrations (see Fig. 4.47).

The deviation between location-specific intercepts and the population-level intercept gives some insight into how much variability may exist in y-intercept values at unsampled locations. For total zinc, half of the sampled watershed catchments overlap well with the population-level intercept. Variability around the population-level intercept and around group-specific random effects (reporting agency and location) is captured in the credible intervals shown in Figure 4.49, as well as in sub-basin credible intervals presented on the Stormwater Heatmap. The Stormwater Heatmap utilizes only the population-level intercept to estimate mean total zinc concentrations at unsampled locations; Figure 4.49 is provided as a visual for how much variability exists between locations, in terms of location-specific intercept.

Figure 4.49 Density plots for the 14 location-specific intercept parameters, along with the median (thick vertical line) and 95% credible intervals (dashed vertical lines) for the population-level intercept. For each location, the mean intercept value is indicated with a black dot, and credible intervals are shown with thick (50% CI) and thin (95% CI) black horizontal lines. Density plots for each location are shaded blue for values that fall within the 95% CI for the population-level intercept, and gray for values that fall outside of the 95% CI for population-level intercept. Note that the population-level 95% CI does not incorporate variability around reporting agency σagency, or around location within each reporting agency σlocation; it is solely the 95% CI for the population-level intercept (i.e.: 95% of the time, the population-level intercept will fall between the given CI values).

In summary, the Stormwater Heatmap utilizes the spatial predictors not_greenBE and sqrt_CO2_transport to estimate the over-annual mean total zinc concentration. The equation used for visualizing the total zinc layer is:

ln(Zn)=4.07+0.42*not_greenBE+0.45*sqrt_CO2_transport

See Applying Mixed Effects Equations to the Stormwater Heatmap at the end of this document for more information on how the Bayesian model is simplified to this equation.

### Total Kjeldahl Nitrogen

Out of 421 Total Kjeldahl nitrogen (TKN) samples, 41 samples (9.7%) were flagged as non-detects, with a variety of detection limits ranging from 30 to 500. The majority of non-detect samples came from three agencies: Pierce County, Port of Tacoma, and Snohomish County (Fig. 4.50). High detection limits in Pierce County and Port of Tacoma strongly truncated the lower half of these data sets. One non-detect sample from TAC_HDR was flagged by the laboratory as estimated, with an estimated value of 255. For the purposes of our analysis, we assume that this estimated sample value is correct. The two samples with the lowest detection limits (one each from TAC_COM and TAC_IND) appear to be anomalous with the rest of the data for these two catchments. Because their presence in the data does not change any of the model outcomes when censored statistics methods are employed, we left them in place. However, their TKN values are in line with what would be expected from mislabeled sample blanks.

Figure 4.50 Concentrations of total Kjeldahl nitrogen in samples collected by catchment location. Red dots indicate detected values, while turqoise dots indicate the detection limit for samples flagged as non-detect.

Although the EPA allows substitution of (0.5 x detection limit) for censored values when less than 15% of samples are non-detect (USEPA, 2009), substitution can add a pattern to the data that is different than the pattern of the data itself (Helsel, 2012). As a result, we performed regression on order statistics (ROS) at each location using the NADA package (Lee, 2020). ROS uses the uncensored and censored values to model the likely data distribution, then draws from the distribution to generate modeled values for the censored data. Because we ran ROS on a location-by-location basis, we do not anticipate the introduction of extraneous patterns that affect the spatial predictors. Rainfall and seasonality patterns could potentially be affected for the 40 data points that were generated through ROS; however, because >90% of data were not censored, this effect should be relatively small. Figure 4.51 shows the modeled concentrations of TKN in turquoise. Frequentist statistics were performed using the modeled values generated by ROS, in place of censored data. Bayesian statistics were performed using a censored data package function the model fitting algorithm (see Bayesian Model Statistics section below).

Figure 4.51 Actual (red) and modeled (turquoise) concentrations of total Kjeldahl nitrogen in samples collected by catchment location. Modeled values were generated by regression on order statistics, to replace the detection limit on censored data.

Based on linear models of ln-transformed TKN versus individual predictors, the strong predictors identified for TKN include: roofs, nodev, sqrt_traffic, sqrt_popn, sqrt_CO2_res, sqrt_CO2_road, sqrt_CO2_almostTotal, devAge2, roof_inURB and roof_intURB_IND (Fig. 4.52).

Figure 4.52 Strong predictors for total Kjeldahl nitrogen (TKN), showing linear model fit (blue line) for the relationship between ln-transformed TKN concentration and each predictor in turn.

The precipitation predictor used for TKN was 14-day cumulative precipitation. In addition, evidence of higher TKN concentrations during summer led us to add summer as a categorical predictor to the TKN model (where summer = 1 during July, August, September, and summer = 0 for all other months).

Residuals plotted against fitted values showed signs of slight heterogeneity (Fig. 4.53, top left plot). Of the variance structures tested, the best fit allows residual variation to differ by reporting agency k.

sd(ϵk)=σk

Figure 4.53 Normalized residuals from beyond-optimal model, with no variance structure (top left), with reporting agency as a variance covariate (top right), and with location as a variance covariate (bottom left). Reporting agency as the variance covariate provided the best model fit.

The best model for TKN is a random-intercept model, where the intercept of the linear model is allowed to vary by reporting agency (k), as well as by location within each reporting agency (j|k). No signs of temporal or spatial auto-correlation were detected in auto-correlation plots or variograms.

Three possible models emerged to capture the fixed effects:

ln(TKNijk)=β0+β1*raini+β2*summer+β3*sqrt_trafficijk+β4*devAge2ijk+ak+aj|k+ϵijk

ln(TKNijk)=β0+β1*raini+β2*summer+β3*sqrt_CO2_roadijk+β4*devAge2ijk+ak+aj|k+ϵijk

ln(TKNijk)=β0+β1*raini+β2*summer+β3*sqrt_trafficijk+β4*nodevijk+ak+aj|k+ϵijk

Each of these models captures two major contributors to total nitrogen: vehicles, and amount/age of development. The AIC scores for all three models were relatively close (796.6, 799.7, 801.4, respectively, when models were fitted using ML).

We selected the first model, with spatial predictors sqrt_traffic + devAge2. While all models fit the data reasonably well, we selected the traffic predictor over on-road CO2 due to the higher spatial resolution of the available data, and the lower AIC value. Furthermore, devAge2 was a stronger predictor in the first model (p-value = 0.006), compared to nodev in the third model (p-value = 0.032). Figure 4.54 shows the model fit for each individual predictor, plotted against data points. Correlation between the two predictors was low (correlation coefficient = 0.4).

Figure 4.54 Single-predictor plots for total Kjeldahl nitrogen, showing fit of the Spatial Predictor Model to each predictor. Model fitting was performed using maximum likelihood (ML) estimation.

Comparisons between the Null Model, Categorical Land Use Model, and Spatial Predictor Model can be visualized through residuals (Fig. 4.55) and comparative metrics such as AIC. The residual plots clearly show good fit (residuals close to zero) for the Categorical Land Use Model and for the Spatial Predictor Model.

Figure 4.55 Total Kjeldahl nitrogen model residuals for the Null Model, Categorical Land Use Model, and Spatial Predictor Model. Each bar represents one watershed, with colors representing agencies. Model fitting was performed using maximum likelihood (ML) estimation.

Final frequentist model AIC values (fitted with REML) for the three models are: 1083.8 (Null Model), 819.4 (Categorical Land Use Model) and 815.5 (Spatial Predictor Model), with the Spatial Predictor Model showing an improvement over the Categorical Land Use Model. Final coefficient values for all three frequentist models are shown in Figure 4.56. The relatively large variability in model coefficients for the Categorical Land Use Model (shown in blue) indicates lower predictive power, because confidence intervals around TKN estimates will be large. We selected the Spatial Predictor Model to estimate COC loading in the Stormwater Heatmap’s data visualizations for its clear improvement in AIC value, and the increase in predictive power over the Categorical Land Use Model.

Figure 4.56 Model coefficients for the Null Model (green), Categorical Land Use Model (blue), and Spatial Predictor Model (red). Final coefficient values for linear mixed effects models are based on fitting with restricted maximum likelihood (REML) estimation, and may differ slightly from those fitted using maximum likelihood (ML) estimation.

#### Bayesian Model Summary

The Bayesian model for TKN was constructed using the censored data function within the brms package (Buerkner, 2017). The best fit model follows a Student-t distribution with the form:

ln(TKNijk)=β0+β1*raini+β2*summeri+β3*sqrt_trafficijk+β4*devAge2ijk+ak+aj|k+ϵijk

where:

β0=6.61;   β1=−0.17;   β2=0.58;   β3=0.23;   β4=0.22

ak∼N(0,σagency);   σagency=0.18

aj|k∼N(0,σlocation);   σlocation=0.22

ϵijk∼t3.35(0,σk);   ln(σk)=η+uk;   σk=eη*euk;   η=−0.80;

uk∼N(0,σ0);   σ0=0.58

Rain is 14-day cumulative precipitation, standardized; and summer is a factor with value = 1 for July, August, September, and value = 0 for all other months. Both spatial predictors (traffic and development age) were first transformed then standardized prior to use. Subscripts indicate sample (i), location (j) and reporting agency (k).

Random effects ak (agency-specific contribution to the intercept) and aj|k (location-specific contribution to the intercept) follow a normal (Gaussian) distribution, with mean equal to 0 and standard deviation equal to σagency for agency-specific contribution to the intercept, and σlocation for location-specific contribution to the intercept. The residual variance ϵijk represents remaining variation in the data that cannot be explained by the model variables. For the TKN model, residual variance follows a Student-t distribution, which is defined by three parameters: shape (ν=3.35) , mean (μ=0) and scale (σ=σj). By imposing a variance structure with reporting agency (k) as the variance covariate, scale (σk) becomes a function of parameters η and uk, where eη represents population-level variability, and euk represents deviation of reporting agency k from the population-level variability. Finally, uk is normally distributed, with mean equal to 0 and standard deviation equal to σ0.

Density plots for the 4000 sets of parameter estimates generated by the Bayesian model are provided in the left half of Figs. 4.57 and 4.58. The peaks of a density plot show where the most likely values (along the x-axis) of a given parameter are concentrated, similar to a histogram. They also indicate how much variability exists around parameter estimates, shown by the shape of the curve (low and broad equals lots of variability; tall and narrow equals little variability). The y-axis on density plots represents the probability density function for the kernel density estimation, and can be disregarded for our purposes. Traces (right half of Figs. 4.57 and 4.58) indicate how well the parameter space was sampled in the four chains generated by the NUTS sampler (no U-turn sampling - an extension of Hamilton Monte Carlo) used when fitting brms models. Each chain generated 1000 sets of parameter estimates, for a total of 4000 sets of parameter estimates. For TKN, the traces indicate that the sampling algorithm did not get stuck in one region of the parameter space, and that the parameter space was well sampled.

Figure 4.57 Posterior densities (left) and traces (right) for parameters associated with fixed effects, obtained from the best-fit Bayesian model for total Kjeldahl nitrogen. In the plots, b_intercept corresponds to β0, b_rain corresponds to β1, b_summer1 corresponds to β2, b_sqrt_traffic corresponds to β3, and b_devAge2 corresponds to β4.

Figure 4.58 Posterior densities (left) and traces (right) for parameters associated with random effects, obtained from the best-fit Bayesian model for total Kjeldahl nitrogen. Here, b_sigma_intercept represents η from the random effects equations, sd_agency__Intercept represents σagency, sd_agency:location__Intercept represents σlocation, sd_agency__sigma_Intercept represents σ0, and nu is the Greek letter ν, representing shape of the Student-t distribution.

Fit of the Bayesian TKN model to data can be assessed through the precision metrics RMSE (0.685) or RSR (0.786), or visualized in estimated versus observed plots (Figs. 4.59 and 4.60). We provide RSR as a measure of model precision, since RSR is a standardized form of RMSE. Moriasi et al. (2007) suggest that an RSR ≤ 0.70 indicates a satisfactory model, while RSR > 0.7 indicates an unsatisfactory model. According to these guidelines, the TKN model (RSR=0.786) is an unsatisfactory model. However, two abnormally low non-detect data points (one at TAC_COM, one at TAC_IND; Fig. 4.50) contribute to the high RSR value for TKN. Removing these two abnormally low TKN observations reduces the RSR to 0.744. Additionally, the high variability in TKN values at the three Snohomish locations (Fig. 4.50 showing reporting limit for censored data; Fig. 4.51 showing the values generated by regression on order statistics) result in a greater spread of residual values (blue points in Fig. 4.59), and hence a higher RSR, for the TKN model.

In figures 4.59 and 4.60, the estimated ln(TKN) values (vertical axis) are generated using the fitted function (brms package; Buerkner 2017), which applies the spatial predictors associated with each data point (location, rain, summer, sqrt_traffic, devAge2) to all 4000 parameter sets that make up the Bayesian posterior. Mean estimated values, along with 95% quantiles, are then plotted against observed ln(TKN) data (horizontal axis). In Figures 4.59 and 4.60, the observed values for censored (non-detect) data are plotted at their detection limit as colored triangles. Ideally, the plotted points should form a swath along the 1:1 diagonal dashed line. This 1:1 line shows where estimated values exactly match the observed data.

For TKN, the estimated vs. observed plots show reasonable model fit over all observed TKN concentrations (Fig. 4.59). Several catchment locations had high variability in TKN concentration (e.g., SNO_COM, SNO_HDR and SNO_LDR). While these catchments had low to medium values for traffic and devAge2 (Fig. 4.60, top panels), the data do not offer a compelling indication that low traffic or devAge2 values are correlated with high variability in observed TKN. In fact, many other locations with low to medium values of traffic and devAge2 (e.g., POT_COM, KIC_LDR) have similar levels of variability in observed TKN as locations with high values of traffic and devAge2 (e.g., SEA_COM).

Summer and rain do not display a consistent effect on the Bayesian model’s fit to observed data (Fig. 4.60, bottom panels).

Figure 4.59 Estimated vs. observed plot for the Bayesian TKN model, with point color corresponding to location, and point style corresponding to whether data were censored (triangles) or not (circles). Censored data are shown at their detection limit. Individual points show how the observed value for ln(TKN) (horizontal axis) compares to the mean estimated value from the Bayesian model (vertical axis). Thin gray vertical lines associated with each point provide the 95% quantiles for each estimate. Note that residual error is excluded from the estimates, resulting in narrower 95% quantiles. The diagonal dashed line shows the 1:1 relationship, where estimated=observed. Note that the two purple triangles farthest to the left correspond to two anomalously low non-detect values from TAC_IND and TAC_COM catchments.

Figure 4.60 Estimated vs. observed plots for the Bayesian TKN model, with colors showing the magnitude of one spatial predictor (sqrt_traffic, devAge2, rain or summer) at each data point, and point style corresponding to whether data were censored (triangles) or not (circles). Censored data are shown at their detection limit. Individual points show how the observed value for ln(TKN) compares to the mean estimated value from the Bayesian model. Diagonal dashed lines show the 1:1 relationship, where estimated=observed. Note that the two triangles farthest to the left correspond to two anomalously low non-detect values from TAC_IND and TAC_COM catchments.

TKN concentration estimates rely on two spatial predictors: sqrt_traffic and devAge2, along with two spatio-temporal predictors: precipitation and season. We can visualize the effect of one spatial landscape predictor at a time on expected TKN concentrations (Fig. 4.61). In this figure, expected mean TKN concentrations as a function of traffic or development age are presented as colored credible interval bands, with S8 Data—used in fitting the Bayesian model—plotted as black points atop the credible intervals. In order to generate Figure 4.62, we integrated over the range of values in the S8 Data for rainfall, season, and the non-displayed spatial predictor, to produce 50%, 80% and 95% credible interval bands. For example, the left half of Figure 4.61 shows expected mean TKN concentrations as a function of traffic, given the observed range of values for development age, rainfall and season.

It is important to point out that Figure 4.61 is not showing the Stormwater Heatmap credible intervals by sub-basin. Stormwater Heatmap credible intervals pertain to a specific sub-basin with known levels of sqrt_traffic and devAge2, thus are expected to be tighter than the ones shown in Figure 4.61, which integrate over a range of values for the non-displayed spatial predictor.

Figure 4.61 credible intervals were calculated using the epred_draws function (tidybayes package; Kay, 2022) applied to a simulated dataset with all combinations of the following predictors:

- 20% of rain & summer combinations randomly sampled from the S8 Data, to provide representative scenarios of rain and summer/ not summer conditions;

- eight equally-spaced sqrt_traffic values between minimum and maximum sqrt_traffic levels observed in the S8 Data;

- eight equally-spaced devAge2 values between the minimum and maximum devAge2 levels observed in the S8 Data.

Following computation of credible intervals, spatial predictors were unstandardized and back-transformed to generate the figure.

As discussed in Step 9 of Model Construction and Selection, credible intervals account for uncertainty around fixed effect parameters and around the variance parameters of the groups, but do not account for residual variance. This allows stormwater managers to assess the following:

- What is the likely range for mean TKN at a heretofore unsampled watershed with a particular level of traffic (or development age)?

This is in line with stormwater management best practices, which manage for the over-annual impact on water bodies rather than for extreme events.

Figure 4.61 Credible intervals for total Kjeldahl nitrogen as a function of raw traffic (Annual Average Daily Traffic; left plot) and development age (right plot). Bands of color represent 50%, 80% and 95% credible intervals, while black points represent S8 Data that were used in fitting the Bayesian model. In this figure, censored data have been replaced with values generated by regression on order statistics. Expected TKN concentrations as a function of traffic (left plot) integrate over rainfall, season, and development age. Expected TKN concentrations as a function of development age (right plot) integrate over rainfall, season and traffic.

The Bayesian TKN model has a population-level intercept β0=6.61. As a random-intercept model, the location-specific intercept is allowed to vary by reporting agency (k), and by catchment location within each reporting agency (j|k), and sums to: β0+ak+aj|k. Density plots for the location-specific intercept parameters are shown in Figure 4.62, along with the mean and 95% credible intervals for the population-level intercept.

Eleven of the 14 catchments have location-specific intercepts that overlap well with the population-level intercept: the majority of their density plots are shaded blue (see e.g., TAC_IND). The remaining three catchments have at least half of their density plot overlapping with that of the population-level intercept (see TAC_COM, SEA_IND, KIC_HDR). The deviation between location-specific intercepts and the population-level intercept gives some insight into how much variability may exist in y-intercept values at unsampled locations. For TKN, all sampled catchments have a y-intercept value that is relatively consistent with that of the population. Variability around the population-level intercept and around group-specific random effects (reporting agency and location) is captured in the credible intervals shown in Figure 4.61, as well as in sub-basin credible intervals presented on the Stormwater Heatmap.

The Stormwater Heatmap utilizes only the population-level intercept to estimate mean TKN concentrations at unsampled locations; Figure 4.62 is provided as a visual for how much variability exists between locations, in terms of location-specific intercept.

Figure 4.62 Density plots for the 14 location-specific intercept parameters, along with the median (thick vertical line) and 95% credible intervals (dashed vertical lines) for the population-level intercept. For each location, the mean intercept value is indicated with a black dot, and credible intervals are shown with thick (50% CI) and thin (95% CI) black horizontal lines. Density plots for each location are shaded blue for values that fall within the 95% CI for the population-level intercept, and gray for values that fall outside of the 95% CI for population-level intercept. Note that the population-level 95% CI does not incorporate variability around reporting agency σagency, or around location within each reporting agency σlocation; it is solely the 95% CI for the population-level intercept (i.e.: 95% of the time, the population-level intercept will fall between the given CI values).

In summary, the Stormwater Heatmap utilizes the spatial predictors sqrt_traffic and devAge2 to estimate the over-annual mean TKN concentration. The equation used for visualizing the total Kjeldahl nitrogen layer is:

ln(TKN)=6.61+0.23*sqrt_traffic+0.22*devAge2

See Applying Mixed Effects Equations to the Stormwater Heatmap at the end of this document for more information on how the Bayesian model is simplified to this equation.

## Model Sensitivity Analysis

We evaluated the model sensitivity to three aspects of the model: 1.) influential data points, 2.) variance structure (no variance covariate, agency as variance covariate, and location as variance covariate), and 3.) structure of the random effects (location only vs. location nested within agency). Alternate models were constructed and compared to the best fit model to evaluate the percent change to landscape predictor parameters. This indicated how sensitive the models were to model structure and influential data points.

We assessed for the presence of influential data points using Cook’s distance, calculated using the HLMdiag package in R (Loy & Hofmann, 2014). Table 4.4 shows the percent difference in landscape predictor coefficients when influential data points were removed for each COC. The effect of influential data points on the models was minimal, with the greatest effect shown for TKN: 5.8% change to sqrt_traffic coefficient when 2 influential data points were removed. One of the two influential data points removed for TKN was randomly generated during model fitting, as the original data point was non-detect.

Table 4.4. Effect of removing influential data points on landscape predictor coefficients and on the global intercept. Columns titled global intercept, sqrt_traffic, devAge2, sqrt_CO2_road, sqrt_CO2_transport and not_greenBE indicate the percent change to these values in models that exclude influential data points compared to models that include influential data points.

We measured the sensitivity of the models to variance structures by running models for each COC with three different variance structures (no variance covariate, agency as variance covariate, location as variance covariate). Landscape predictor and global intercept coefficients generated by these models were compared to those of the best fit model for each COC, to obtain the percent difference in coefficients (Table 4.5). Substituting different variance structures into the equations had little impact on the global intercept and landscape predictor coefficients. The maximum percent difference of 4.9% was seen for total zinc when using no variance covariate compared with the best fit model (agency as variance covariate).

Table 4.5. Effect of changing the model variance structure on landscape predictor coefficients and on the global intercept. The best fit variance covariate and comparison covariate columns indicate which variance covariate is used in the best fit model and the covariate structure that is being tested as a comparison. Columns titled global intercept, sqrt_traffic, devAge2, sqrt_CO2_road, sqrt_CO2_transport and not_greenBE indicate the percent change to these values in models where the variance structure has been changed.

Lastly, we evaluated how sensitive the SWHM models are to the choice of using a nested structure for the random effects (location nested within agency) rather than a non-nested structure, with location as the sole random effect. We ran models using location as the sole random effects, comparing the landscape predictor and global intercept coefficients to those from the best fit (nested random effects) models, to obtain the percent difference in coefficients for each COC (Table 4.6). Altering the random effects structure had a larger impact on coefficients than did the previous sensitivity analyses (removing influential outliers or changing variance structure). The greatest percent change was for Total Phosphorus, where removing the nested structure of the data led to a 27% change in the coefficient for sqrt_CO2_road.

Table 4.6. Effect of changing the random effects stucture (from location nested within agency to not nested) on landscape predictor coefficients and the global intercept. Columns titled global intercept, sqrt_traffic, devAge2, sqrt_CO2_road, sqrt_CO2_transport and not_greenBE indicate the percent change to these values in models where the random effect of location is NOT nested in within agency.

## Model Validation with External Data

We performed model validation using nine independent data sets (“validation data”) to determine whether the model results are representative of locations outside of the 14 catchments used to fit and calibrate the models. This is an important step for ensuring that our chemical of concern models can be applied to areas across the Puget Sound region.

The validation process involved several steps:

### Step 1: Identify suitable data

We searched for projects where data were collected in a similar way to the S8 data used for model fitting. The S8 data were collected from stormwater outfalls as flow-composite samples during storms with predicted cumulative rainfall equal to or exceeding 0.25 inches. Ideal validation data would share these sample characteristics. Because of the low number of suitable studies with flow-composite samples, we also included outfalls with grab samples that were collected on three or more occasions over time. Initially, we treated the grab sampled outfalls differently in the model validation process, as variability around grab sample results can be substantially higher than variability around composite samples. However, as we did not see evidence of higher variability in the grab-sampled catchments that we had identified, we ultimately combined grab-sampled outfalls with those sampled using flow-composite methodology.

### Step 2: Prepare spatial predictors for validation catchments

We obtained or traced spatial polygons for each validation catchments. Using these polygons, we convolved then calculated the mean spatial predictor values for each validation catchment.

Convolved mean spatial predictor values for each catchment were standardized using the spatial predictor means and standard deviations from the S8 fitting dataset. Standardized spatial predictors were capped at +/- 2 to prevent extrapolation of model equations beyond the extent of the fitting data.

### Step 3: Prepare the validation data

We removed validation data that were collected on days when the Daymet daily surface weather dataset (Thornton et al, 2022) estimated cumulative daily rainfall at 0mm.

We also used Cleaveland dotplots of the ln-transformed stormwater validation data to identify and remove outliers. Two high outliers were removed for zinc (both at OF235), one high outlier was removed for TKN (at SR9-MP17.92), one high outlier (at SS4) and two low outliers were removed for TSS (one at OF235, one at OF237A), and one high outlier was removed for phosphorus (at SR9-MP17.92). The two low outliers for TSS occurred on days with very low rainfall (Daymet precipitation < 1mm).

### Step 4: Obtain prediction intervals and fitted values for validation catchments

Using the posterior_predict and fitted function in the brms package (Buerkner, 2017), we applied the Bayesian water quality models to the spatio-temporal predictors (landscape predictors, season, Daymet cumulative rainfall) from the validation catchments to obtain prediction intervals and fitted values. Prediction intervals predict in what range a future individual observation will fall. With the nested model structure for random effects (location nested within agency), prediction intervals are able to make predictions about future samples both for known agencies and unknown (new) agencies. This makes them ideal for comparison to validation data. In the case of the Tacoma S8C validation dataset, these data were collected by a known agency with an agency-specific intercept (City of Tacoma), so the prediction intervals are tighter for Tacoma S8C validation data than for validation data from unknown agencies.

Fitted values provide a direct comparison between individual samples collected from validation catchments and model estimates for that location and weather conditions (rainfall, season).

### Step 5: Compare validation data to model predictions

We plotted the validation data against prediction intervals and fitted values to visually assess whether validation data fall within a margin of the model predictions. The following tools were used for visual assessment:

- Graphical evaluation of validation data plotted atop prediction interval bands. We were looking to see if validation data generally fit within the 50%, 80% or 95% prediction intervals. This would allow us to conclude that the model is representative of the Puget Sound basin.

- Graphical evaluation of validation data plotted against model predictions (observed versus estimated plots). We were looking for validation data to fall relatively close to the 1:1 observed vs. estimated line.

- Graphical observation of explanatory reasons for divergence between model predictions and validation data.

### Validation Datasets

All validation data were collected between 2004 and 2018. Five composite data sets and four grab sample data sets were utilized for model validation, for a total of 28 catchments (Table 4.7). The sampled catchments span a relatively wide range of the Puget Sound region, with different land use characteristics (Fig. 4.63). Twenty catchments are located in urban areas, while eight are located in medium- to low-density residential areas outside of the Seattle-Tacoma urban corridor. Of the twenty urban catchments, nine are on high-use roadways (MLK Street Sweeping project, Washington State DOT project). Three of the catchments also appear in the S8 fitting data (OF235, OF237B and OF245), and were resampled by the city of Tacoma for their S8C study.

Table 4.7. Sample locations for validation data, including sample type (composite or grab), location description, and whether the samples were tested for COCs of interest (Cu=total copper, Zn=total zinc, TSS=total suspended solids, P=total phosphorus, TKN=total Kjeldahl nitrogen).

Figure 4.63 Locations of test data outfalls used in model validation. Garden Ed and Lahti Drive outfalls are proximally located and are combined into one point on the map.

### Validation Catchments - Spatial Predictors

One way of assessing the similarity of the model validation catchments to the S8 model fitting catchments is to compare the standardized spatial predictor values for the fitting vs validation catchments. Standardized spatial predictor values for the validation catchments were similar to those of the fitting catchments for sqrt_CO2_road, devAge2 and not_greenBE. All of these range roughly between +/-2 (Fig. 4.64). In contrast, the sqrt_traffic predictor is substantially higher than this range for a number of the validation catchments (Fig. 4.64). The S8 fitting data did not have any purely roadway catchments, whereas nine of the validation catchments (those from the MLK Street Sweeping and WA State DOT projects) are paved roadway with high levels of traffic.

Figure 4.64 Standardized spatial predictor values for sqrt_CO2_road, (purple) sqrt_traffic (red), devAge2 (orange) and not_greenBE (navy) at the validation catchments. Dashed lines at y=-2 and y=2 show the values at which predictors were capped.

As discussed above in “Step 2: Prepare spatial predictors for validation catchments”, all standardized spatial predictors for the validation catchments were capped at +/- 2 to keep model predictions within the range represented by the fitting data. Based on the validation results, this capping value appears appropriate — predictions of stormwater contaminants from fully-paved, high-traffic catchments would be too high if the traffic predictor was not capped. It is possible that the sqrt_traffic predictor is only linear over a certain range of traffic values, and that the relationship between chemical concentration and sqrt_traffic levels becomes nonlinear at traffic values outside of those seen in the S8 fitting data.

### Validation Results

#### Total Copper

Six of the nine model validation projects (22 catchments total) analyzed stormwater samples for total copper. These copper data are plotted atop prediction intervals for their catchments (Fig. 4.65). Prediction intervals show the range into which future samples might fall. In general, our copper model tended to over-estimate copper concentrations in the validation catchments, with the exception of several highway sites (catchments beginning with I5) and the sites in Coupeville (BoatLaunch, CoupevilleWharf, FrontAndMain): the majority of samples from these catchments fell within the 50% prediction intervals.

Tacoma S8C outfalls (samples beginning with OF) have somewhat narrower prediction intervals because the agency (the random effect) has a known agency-specific intercept from the model fitting process.

Figure 4.65 Box and whisker plots for copper validation data, shown atop model-generated prediction intervals. Dark orange bands represent the 50% prediction intervals, where 50% of future data points would be expected to lie, light orange bands represent 80% prediction intervals, and peach bands represent 95% prediction intervals.

Estimated vs. observed plots show the model estimated values for each catchment plotted against observed validation data (Fig. 4.66, 4.67 and 4.68). Figure 4.66 shows all observed data points and their model estimated values. Figure 4.67 shows a series of estimated vs. observed plots where colors indicate the spatio-temporal predictor values associated with each catchment and sample. Lastly, Figure 4.68 shows the median estimated value vs. the median observed value as a summary of the data for each catchment. Ideally, data points should lie close to the 1:1 line indicating that model estimates are close to the observed values. For copper, many data points have higher estimated values than the actual observed values.

Several samples from the Tacoma outfalls (OF235, OF237A, OF243) had estimated values substantially higher than their observed values (see upper left, Fig. 4.66). These samples were collected in the summer during very low rainfall events (Fig. 4.67, bottom panels). Two explanations exist for this overestimation. It is possible that the rain and summer coefficients in our copper model are not suitably addressing low rainfall/ summer conditions. It is also possible that the level of rainfall was too low to convey copper particles to the stormwater outfall, resulting in less polluted water entering the outfall during these sampling events.

Figure 4.66 Estimated vs. observed plot for the copper validation data, with point color and shape corresponding to catchment location. Individual points show how the observed value for ln(copper) from model validation catchments (horizontal axis) compares to the mean estimated value from the Bayesian model for those same catchments (vertical axis). The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

Figure 4.67 Estimated vs. observed plots for the copper validation data, with colors showing the magnitude of one spatial or temporal predictor (sqrt_traffic, devAge2, rain, summer) at each data point. Individual points show how the observed values for ln(copper) at the model validation catchments compare to the mean estimated values from the Bayesian model for the same catchments. Diagonal dashed lines show the 1:1 relationship, where estimated=observed.

Figure 4.68 Median estimated vs. median observed plots for total copper at validation catchments. The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

#### Total Suspended Solids

Seven of the nine model validation projects (25 catchments total) analyzed stormwater samples for total suspended solids. These TSS data are plotted atop prediction intervals for their catchments (Fig. 4.69). Prediction intervals show the range into which future samples might fall. In general, the 50% prediction intervals (dark orange, Fig. 4.69) overlap well with validation data. For most catchments, the 95% prediction intervals extend far beyond the range of the data points, indicating that the S8 fitting data may have had higher variability around TSS data points than the validation data.

Tacoma S8C outfalls (samples beginning with OF) have somewhat narrower prediction intervals because the agency (the random effect) has a known agency-specific intercept from the model fitting process.

Figure 4.69 Box and whisker plots for TSS validation data, shown atop model-generated prediction intervals. Dark orange bands represent the 50% prediction intervals, where 50% of future data points would be expected to lie, light orange bands represent 80% prediction intervals, and peach bands represent 95% prediction intervals.

Estimated vs. observed plots show the model estimated values for each catchment plotted against observed validation data (Fig. 4.70, 4.71 and 4.72). Figure 4.70 shows all observed data points and their model estimated values. Figure 4.71 shows a series of estimated vs. observed plots where colors indicate the spatio-temporal predictor values associated with each catchment and sample. Lastly, Figure 4.72 shows the median estimated value vs. the median observed value as a summary of the data for each catchment. Ideally, data points should lie close to the 1:1 line indicating that model estimates are close to the observed values. For TSS, the validation data points are arranged in a cloud centered around the 1:1 line (Fig. 4.70). Figure 4.72 shows that our model is overestimating TSS values for five outfalls from two agencies (KIR-CW, N.MER, REN-JC, HS-033, NB-020).

The subplots in Figure 4.71 do not indicate a strong pattern of correlation between data points farther from the 1:1 line and magnitude of the spatio-temporal predictors. The rainfall plot (Fig. 4.71, lower left) indicates a possible correlation between low 1-day cumulative rainfall and greater distance between data points and the 1:1 line. Light rainfall may mobilize less particulates on the ground, leading to lower observed TSS than the model predicts.

Figure 4.70 Estimated vs. observed plot for the TSS validation data, with point color and shape corresponding to catchment location. Individual points show how the observed value for ln(TSS) from model validation catchments (horizontal axis) compares to the mean estimated value from the Bayesian model for those same catchments (vertical axis). The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

Figure 4.71 Estimated vs. observed plots for the TSS validation data, with colors showing the magnitude of one spatial or temporal predictor (sqrt_traffic, devAge2, rain) at each data point. Individual points show how the observed values for ln(TSS) at the model validation catchments compare to the mean estimated values from the Bayesian model for the same catchments. Diagonal dashed lines show the 1:1 relationship, where estimated=observed.

Figure 4.72 Median estimated vs. median observed plots for TSS at validation catchments. The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

#### Total Phosphorus

Six of the nine model validation projects (20 catchments total) analyzed stormwater samples for total phosphorus. The phosphorus data are plotted atop prediction intervals for their catchments (Fig. 4.73). Prediction intervals show the range into which future samples might fall. In general, the 50% prediction intervals (dark orange, Fig. 4.73) overlap well with validation data. Several catchments had higher phosphorus concentrations than the model predicted (OF243, SS3, SS5).

Tacoma S8C outfalls (samples beginning with OF) have slightly narrower prediction intervals because the agency (the random effect) has a known agency-specific intercept from the model fitting process.

Figure 4.73 Box and whisker plots for total phosphorus validation data, shown atop model-generated prediction intervals. Two data points extended significantly beyond the scope of the plot and are not shown here (OF243: 2430ug/l and 3680 ug/l). Dark orange bands represent the 50% prediction intervals, where 50% of future data points would be expected to lie, light orange bands represent 80% prediction intervals, and peach bands represent 95% prediction intervals.

Estimated vs. observed plots show the model estimated values for each catchment plotted against observed validation data (Fig. 4.74, 4.75 and 4.76). Figure 4.74 shows all observed data points and their model estimated values. Figure 4.75 shows a series of estimated vs. observed plots where colors indicate the spatio-temporal predictor values associated with each catchment and sample. Lastly, Figure 4.76 shows the median estimated value vs. the median observed value as a summary of the data for each catchment. Ideally, data points should lie close to the 1:1 line indicating that model estimates are close to the observed values. For total phosphorus, a few outfalls have the median of their points below the 1:1 line (OF243 and SS3, SS5), with our model underestimating phosphorus values in these catchments (Fig. 4.76).

The subplots in Figure 4.75 do not indicate any pattern between data points farther from the 1:1 line and magnitude of the spatio-temporal predictors. It may be that some of the validation locations display high variability because of an unaccounted for temporal predictor.

Figure 4.74 Estimated vs. observed plot for total phosphorus validation data, with point color and shape corresponding to catchment location. Individual points show how the observed value for ln(phosphorus) from model validation catchments (horizontal axis) compares to the mean estimated value from the Bayesian model for those same catchments (vertical axis). The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

Figure 4.75 Estimated vs. observed plots for phosphorus validation data, with colors showing the magnitude of one spatial or temporal predictor (sqrt_CO2_road, rain, summer) at each data point. Individual points show how the observed values for ln(phosphorus) at the model validation catchments compare to the mean estimated values from the Bayesian model for the same catchments. Diagonal dashed lines show the 1:1 relationship, where estimated=observed.

Figure 4.76 Median estimated vs. median observed plots for total phosphorus at validation catchments. The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

#### Total Zinc

Six of the nine model validation projects (22 catchments total) analyzed stormwater samples for total zinc. Zinc data are plotted atop prediction intervals for their catchments (Fig. 4.77). Prediction intervals show the range into which future samples might fall. For many catchments, the 95% prediction intervals for zinc extend far above the data points in the validation catchments. The 50% intervals fit well for nearly half of the catchments, while the other half are over-estimated.

Tacoma S8C outfalls (samples beginning with OF) have somewhat narrower prediction intervals because the agency (the random effect) has a known agency-specific intercept from the model fitting process.

Figure 4.77 Box and whisker plots for total zinc validation data, shown atop model-generated prediction intervals. Dark orange bands represent the 50% prediction intervals, where 50% of future data points would be expected to lie, light orange bands represent 80% prediction intervals, and peach bands represent 95% prediction intervals.

Estimated vs. observed plots show the model estimated values for each catchment plotted against observed validation data (Fig. 4.78, 4.79 and 4.80). Figure 4.78 shows all observed data points and their model estimated values. Figure 4.79 shows a series of estimated vs. observed plots where colors indicate the spatio-temporal predictor values associated with each catchment and sample. Lastly, Figure 4.80 shows the median estimated value vs. the median observed value as a summary of the data for each catchment. Ideally, data points should lie close to the 1:1 line indicating that model estimates are close to the observed values. Figure 4.80 shows that our model is overestimating median total zinc for three outfalls (HS-033, OF243 and I5_MP197.27).

The subplots in Figure 4.79 suggest that the samples where our model overestimated zinc values come from high traffic areas (upper right) during times of low rainfall (lower left) and summer (lower right).

Figure 4.78 Estimated vs. observed plot for total zinc validation data, with point color and shape corresponding to catchment location. Individual points show how the observed value for ln(zinc) from model validation catchments (horizontal axis) compares to the mean estimated value from the Bayesian model for those same catchments (vertical axis). The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

Figure 4.79 Estimated vs. observed plots for total zinc validation data, with colors showing the magnitude of one spatial or temporal predictor (not_greenBE, sqrt_traffic, rain, summer) at each data point. Individual points show how the observed values for ln(zinc) at the model validation catchments compare to the mean estimated values from the Bayesian model for the same catchments. Diagonal dashed lines show the 1:1 relationship, where estimated=observed.

Figure 4.80 Median estimated vs. median observed plots for total zinc at validation catchments. The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

#### Total Kjeldahl Nitrogen

Three of the nine model validation projects (10 catchments total) analyzed stormwater samples for total Kjeldahl nitrogen. Unfortunately, all of these catchments are located in high-traffic roadways, with nearly all of the catchments being completely paved. In Figure 4.81 TKN data are plotted atop prediction intervals for their catchments. Prediction intervals show the range into which future samples might fall. The prediction intervals fit well for the relatively small quantity of TKN validation data.

Figure 4.81 Box and whisker plots for TKN validation data, shown atop model-generated prediction intervals. Dark orange bands represent the 50% prediction intervals, where 50% of future data points would be expected to lie, light orange bands represent 80% prediction intervals, and peach bands represent 95% prediction intervals.

Estimated vs. observed plots show the model estimated values for each catchment plotted against observed validation data (Fig. 4.82, 4.83 and 4.84). Figure 4.82 shows all observed data points and their model estimated values. Figure 4.83 shows a series of estimated vs. observed plots where colors indicate the spatio-temporal predictor values associated with each catchment and sample. Lastly, Figure 4.84 shows the median estimated value vs. the median observed value as a summary of the data for each catchment. Ideally, data points should lie close to the 1:1 line indicating that model estimates are close to the observed values. With a few exceptions, most of the available TKN data lie close to the 1:1 line (Fig 4.82).

The scale on the sqrt_traffic subplot of Figure 4.84 (upper left) shows the high traffic nature of the available TKN validation data. We were not able to find data from a more diverse set of landscapes for model validation.

Figure 4.82 Estimated vs. observed plot for the TKN validation data, with point color and shape corresponding to catchment location. Individual points show how the observed value for ln(TKN) from model validation catchments (horizontal axis) compares to the mean estimated value from the Bayesian model for those same catchments (vertical axis). The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

Figure 4.83 Estimated vs. observed plots for TKN validation data, with colors showing the magnitude of one spatial or temporal predictor (sqrt_traffic, devAge2, rain, summer) at each data point. Individual points show how the observed values for ln(TKN) at the model validation catchments compare to the mean estimated values from the Bayesian model for the same catchments. Diagonal dashed lines show the 1:1 relationship, where estimated=observed.

Figure 4.84 Median estimated vs. median observed plots for TKN at validation catchments. The diagonal dashed line shows the 1:1 relationship, where estimated=observed.

## Model Fitting Conclusions

This section provides a summary of model fit for each COC, including sensitivity analysis and model validation. Discussion of water quality model utility on the Stormwater Heatmap is included for each COC.

#### Total Copper

Spatial predictor model: ln (total copper) = f (21-day precip, summer, sqrt_traffic, devAge2)

Primary sources of total copper to urban stormwater runoff in Washington State include wear of vehicle brake pads; building materials such as roofing, treated lumber and siding materials; parking lots; and vehicle exhaust (Bookter 2017). Additionally, total copper in stormwater runoff is related to flow, rainfall intensity and antecedent dry period (Sharma et al., 2016).

The spatial predictor model for total copper is a clear improvement over the land use category model, and has good fit to the S8 fitting data. The spatial predictor model has an AIC value nearly four points lower than that of the categorical land use model as assessed in the frequentist model fitting. Furthermore, error bars around the spatial predictor model covariates were much smaller than error bars around covariates from the land use category model (Fig.4.9).

During the Bayesian model fitting steps, we noted that the spatial predictor model fits well to data from high sqrt_traffic and high devAge 2 (old development) locations (Fig. 4.13). The model seems to over- or under-predict total copper in some of the low-traffic locations from the S8 dataset. Additionally, location-specific intercepts arising from the nested random effects structure of the model showed some variability between locations (Fig. 4.15). This indicates that we might expect total copper levels to vary somewhat from our model predictions in new locations, although the slope of the relationship between ln(Cu) and the scaled and centered spatial predictors should be consistent.

The spatial predictor model for total copper fits the S8 data well and is robust to changes in the model structure. The calculated RSR for the Bayesian total copper model is 0.506, indicating a model on the cusp of good and very good. Removing influential data points (only 2 for copper) or changing the variance structure resulted in <1% change to model coefficients. Removing the nesting structure of the random effects from the model had a moderate effect on model coefficients (8% change in sqrt_traffic coefficient, 3% change in devAge2 coefficient).

Model validation was performed using copper data from 22 outfalls that included busy streets, highway areas, high-density urban or industrial areas, and lower density residential areas. The total copper model generally fits the trends of the validation locations well: areas with high traffic or old development are correctly predicted to have higher copper concentrations than areas with low traffic and/or new/low development (Fig.4.65). However, the model generally over-predicts the range of possible copper values for most outfalls (Fig. 4.65). The model fit data from high traffic locations very well (Fig. 4.67), but over-predicted copper for other areas. Low rainfall frequently coincided with data points that fell outside of the model predictions (Fig. 4.67).

In general, we think the copper model is a solid improvement over land use category models, and can be relied upon to show areas that are likely to have higher magnitudes of total copper in stormwater throughout the Puget Sound Basin. The overall estimates (especially 80 and 95% prediction intervals) may be higher than reality, particularly for low traffic/ newer development watersheds.

#### Total Suspended Solids

Spatial predictor model: ln (TSS) = f (1-day precip, sqrt_traffic, devAge2)

Sources of TSS to stormwater are challenging to quantify, but are generally related to degree of urbanization (Pope & Bevans, 1987). In urban stormwater, sediment source fingerprinting indicates that coarse sediments primarily originate from road deposited sediment, while fine particles originate from soils and storm grate sediments (Zhu et al., 2024). Road-deposited sediment includes anthropogenic sources (pavement wear, vehicle exhaust, tire and vehicle body wear, road salt, road paint, pedestrian debris, building and construction material); and natural sources (soil material, plant and leaf litter, atmospheric deposition) (Taylor & Owens, 2009). Additionally, high rainfall intensity and large amounts of rainfall, along with long antecedent dry periods, can lead to higher concentrations of suspended solids in runoff (Zhu et al., 2024).

The spatial predictor model for TSS is a clear improvement over the land use category model, and has good fit to the S8 fitting data. The spatial predictor model has an AIC value six and a half points lower than that of the categorical land use model as assessed in the frequentist model fitting. Furthermore, error bars around the spatial predictor model covariates were much smaller than error bars around covariates from the land use category model (Fig.4.20).

In the Bayesian model fitting plots, we see a large spread in TSS values at each location (e.g., Fig. 4.23). The rainfall predictor for TSS is 1-day rainfall, while all other COC models have a 14- to 21-day cumulative rainfall predictor. It could be that an additional temporal predictor (season, longer duration cumulative rainfall) might improve model fit for TSS by accurately capturing within-site variability. (In the frequentist model fitting steps, we tested summer as a temporal predictor, but found it to not be significant for TSS.) Location-specific intercepts arising from the nested structure of the model’s random effects showed little variability between locations (Fig. 4.26), indicating that we might expect the TSS model to fit well at new locations.

The calculated RSR for the Bayesian TSS model is 0.81, indicating an unsatisfactory model. However, the large spread in TSS values over time at each location is a clear contributor to the high RSR value. Because the Stormwater Heatmap links chemicals of concern to spatial landscape predictors (rather than temporal ones), we think that the high RSR is not reflective of the model’s utility in this context.

The TSS model is robust to changes in the model structure. Removing influential data points (3 for TSS) or changing the variance structure resulted in a maximum of 3.3% change to model coefficients. Removing the nesting structure from the model had a moderate effect on the sqrt_traffic coefficient (10.5% change), but very little effect on the devAge2 coefficient (1.1% change) and the intercept (0.2% change).

Model validation was performed using TSS data from 25 outfalls that included busy streets, highway areas, high density urban or industrial areas, and low density residential areas. The TSS model fits the validation locations well. The upper bounds of the 95% prediction intervals are well above the range of the TSS validation data, but the 50% prediction intervals are very good at capturing the median, and 80% prediction intervals capture almost all outlying data points (Fig. 4.69). As with the S8 fitting data, the validation catchments also show a broad spread of TSS values over time for each catchment. Low quantities of rainfall may have limited effectiveness at transporting solids to storm drains, resulting in high variability over time, or unexplored seasonal predictors may also have an effect.

In general, we think the TSS model is a solid improvement over land use category models, and can be relied upon to show locations of high TSS in the Puget Sound Basin. The model also has been shown to provide reliable predictions for TSS magnitude at new locations. Large variability in TSS values at each location may be the result of variable effectiveness of precipitation in transporting solids to storm drains, or unexplored seasonal effects, including autumn, winter or spring.

#### Total Phosphorus

Spatial predictor model: ln (total phosphorus) = f (21-day precip, summer, sqrt_CO2_road)

Sources of phosphorus in urban landscapes are heterogeneously distributed and include disparate contributors such as construction activities, fertilizers, street solids, animal waste, grass clippings, municipal wastewater, leaf litter, atmospheric deposition and automobile emissions (Zhou et al., 2026). Furthermore, impervious land cover increases total phosphorus in urban stormwater through decreased inflitration into soil and accelerated surface runoff (Jacobson, 2011; Yang and Lusk, 2018). While there are a few phosphorus sources in stormwater that can be directly linked to road usage (e.g., street solids, automobile emissions), sqrt_CO2_road in our model likely serves as a proxy for human activities that release phosphorus.

The spatial predictor model for total phosphorus offers a solid improvement over the land use category model, and has decent fit to the S8 fitting data. The spatial predictor model has sqrt_CO2_road as the single spatial predictor, along with summer and 21-day cumulative precipitation as temporal predictors. This model has an AIC value over four and a half points lower than that of the categorical land use model as assessed by frequentist model fitting. Interestingly, the sqrt_CO2_road coefficient has a p-value of 0.08 when fit by maximum likelihood, indicating that this predictor may not be necessary for model fit to the S8 data. Removing sqrt_CO2_road as a predictor results in a temporal predictor model with an AIC value one point lower than the spatial predictor model. This is not entirely unexpected, as AIC applies a two-point penalty for every additional parameter. We chose to move forward with the spatial predictor model that includes sqrt_CO2_road, as removing it insinuates that any level of human activity over a landscape has equal impact on total phosphorus concentrations. While error bars around the sqrt_CO2_road predictor were wide enough to extend through 0, the error bars around coefficients from the land use category model were substantially larger, further indicating that the spatial predictor model provides an improvement over the land use category model (Fig. 4.31).

In the Bayesian model fitting plots, we see a large spread in total phosphorus values at outfalls with low sqrt_CO2_road values, and less spread in total phosphorus values at locations with high sqrt_CO2_road values (Fig. 4.35, top left panel). This dichotomy suggests that low-traffic regions may have a greater temporal impact in phosphorus-generating activities than our model was able to capture. Temporally-specific contributors to phosphorus in urban landscapes include fertilizer application, grass clippings and leaf litter. These sources vary in their prevalence by season, which may not be captured by using only summer as a temporal predictor. Another portion of the Bayesian analysis involved examining location-specific intercepts that arise from the nested random effects structure, and comparing them to the population-level intercept. Location-specific intercepts show little variability between locations (Fig. 4.37), indicating that we might expect the intercept for the total phosphorus model to fit well to new locations.

The calculated RSR for the Bayesian total phosphorus model is 0.66, indicating a satisfactory model. The large spread in total phosphorus values at less trafficked outfalls is a clear contributor to the high RSR value. Because the Stormwater Heatmap links chemicals of concern to spatial landscape predictors (rather than temporal ones), we think that the high RSR is not reflective of the model’s utility in this context.

The total phosphorus model is robust to some changes in the model structure. Removing influential data points (only one for total phosphorus) or changing the variance structure had little effect on model coefficients (maximum of 1% change in coefficient values). However, removing the nesting structure from the model’s random effects resulted in a 27% increase in the sqrt_CO2_road coefficient, from 0.30 in the nested model to 0.39 in the non-nested model. From the perspective of model robustness, a shift to a steeper (more significant) slope for this relationship is less concerning than a shift toward a less steep (and less significant) slope. If our model assumption of a nested relationship for location within agency is not correct, there is an even stronger relationship between sqrt_CO2_road and total phosphorus.

Model validation was performed using total phosphorus data from 20 catchments that included busy streets, highway areas, high density urban or industrial areas, and a few low density residential areas. Seven of the 20 catchments had high variability in total phosphorus values, while the other catchments had much less variability (Fig. 4.73). Fifty percent prediction intervals fit the catchments with low variability well, but 80 and 95% prediction intervals overestimate the range of possible phosphorus values. ln contrast, the seven catchments with high variability were generally underestimated by the model (Fig. 4.73). The model validation catchments offer no clear pattern for catchments where model fit is good versus those where it is less good.

Total phosphorus turned out to be a challenging chemical to model in stormwater, perhaps because its main sources in urban areas are from divergent land uses. Lawns and roads contribute the majority of phosphorous load to stormwater, but land use affects the contribution from different sources. Lawns and leaf litter are more important in residential areas, while roads are more important in commercial and industrial areas (Waschbusch et al, 1999). These varied sources proved difficult to model with the data that were available. The unique timing of plant-based phosphorus sources to stormwater (fertilizer application, grass clipping, leaf litter) also led to high variability at monitored locations. While the total phosphorus spatial predictor model is a solid improvement over the land use category model, future modeling efforts would benefit from additional data as well as exploration of new spatial and temporal predictors for total phosphorus. Catchments in agricultural areas should also be included in a future analysis, as agricultural runoff is usually higher than urban runoff in total phosphorus, and the S8 fitting dataset was focused on catchments in urban areas rather than agricultural areas.

#### Total Zinc

Spatial predictor model: ln (total zinc) = f (14-day precip, summer, not_greenBE, sqrt_CO2_transport, rain * not_greenBE)

The largest contributing sources of zinc to stormwater are galvanized metals including roofs, ductwork and fences; and vehicle tire wear (Davis et al, 2001). In the spatial predictor model, the not_greenBE predictor serves as a proxy for developed areas that may be more likely to contain galvanized metals, while the sqrt_CO2_road predictor catches the contribution of vehicle tires to zinc in stormwater.

The spatial predictor model for total zinc offers improvement over the land use category model in terms of reduced uncertainty around coefficients, but is not a clear winner. Model comparison with AIC indicates that the land use category model has better fit to the S8 data than the spatial predictor model by three and a half points. However, error bars around coefficients from the land use category model are substantially larger than those around spatial predictor model coefficients, indicating a large level of uncertainty in the land use category model (Fig. 4.43). This reduces the utility of the land use category model across the Puget Sound basin, and makes us prefer the spatial predictor model, despite its higher AIC value.

In the Bayesian model fitting plots, we see a large spread in total zinc values at several locations (Fig. 4.46), with no pattern detected for which locations have high variability. This may be caused by a combination of two factors: zinc from galvanized metal is heterogeneously distributed across landscapes, and rainfall dynamics can mobilize zinc in different ways. Within a single catchment, stormwater pollutant concentrations can vary based on when samples are collected during a storm, with high concentrations during initial runoff, and intra-event variability as different parts of the catchment contribute runoff at different times. Additionally, storm intensity, duration, and antecedant dry period can affect how much zinc is mobilized into stormwater. Interestingly, sites with high not_greenBE (very developed sites) and sites with high sqrt_CO2_transport had lower variability (Fig. 4.47, top panels).

Another portion of the Bayesian analysis involved examining location-specific intercepts that arise from the nested random effects structure, and comparing them to the population-level intercept. Location-specific intercepts show high variability between locations, with only half of the locations overlapping significantly with the population-level intercept (Fig. 4.49). This indicates that we might expect total zinc levels to vary somewhat from our model predictions in new locations, and that we might expect high variability around prediction intervals for new samples.

The calculated RSR for the Bayesian total zinc model is 0.53, indicating a good model. Despite the large spread in total zinc values at several locations, the spatial predictor model fit the majority of S8 data reasonably well (Fig. 4.46).

The total zinc model is robust to changes in the model structure. Removing influential data points (five for total zinc) had little effect on model coefficients (maximum of 2% change in coefficient values). Changing the model variance structure had a small impact (5% change in sqrt_CO2_transport coefficient), and removing the random effects nesting structure had little impact (up to 1.3% change in coefficient values).

Model validation was performed using total zinc data from 22 catchments that included busy streets, highway areas, high density urban or industrial areas, and several low density residential areas. Prediction intervals generated from the model overestimate zinc concentrations, especially for industrial and high traffic locations (Fig. 4.77). Rainfall and season also seem to play a role: samples where zinc was overestimated had low 14-day cumulative rainfall (Fig. 4.79, bottom left), and the most over-estimated samples occurred in summer in high-traffic areas. This indicates the possibility of low mobilization of zinc to outfalls during collection of some samples.

One of the challenges of fitting zinc models is that a major contributor, galvanized metals, are found in localized hotspots that are difficult to categorize across a landscape. Galvanized materials may be more prevalent in commercial and industrial areas, but residential areas can also be sources of galvanized metal. Currently, there are no remote sensing tools that can distinguish galvanized metal surfaces from non-galvanized surfaces, although this capability may be available in the future. A spatial predictor layer for galvanized surfaces will likely be a game-changer for modeling zinc concentrations in stormwater. For now, the Stormwater Heatmap zinc layer can serve to highlight areas where zinc is likely to have higher concentrations, but numeric estimates may be high.

#### Total Kjeldahl Nitrogen

Spatial predictor model: ln (TKN) = f (14-day precip, summer, sqrt_traffic, devAge2)

TKN represents the sum of total organic nitrogen (nitrogen bound to carbon from living organisms), ammonia and ammonium. Its primary sources into stormwater are from plant debris (Jani et al., 2020), road runoff (Ma et al., 2021), pet waste, chemical fertilizers, leaking sewers and soil erosion (Carey et al., 2013). The nitrogen components of TKN are positively correlated with total rainfall and antecedant dry period (Jani et al, 2020).

During frequentist model fitting, we determined that the spatial predictor model for TKN is a clear improvement over the land use category model, with an AIC value almost four points lower than that of the categorical land use model. Furthermore, error bars around the spatial predictor model covariates were much smaller than error bars around covariates from the land use category model (Fig.4.56). Approximately 10% of samples analyzed for TKN were non-detect. Regression on order statistics (ROS) were used during frequentist model fitting to generate TKN values below the detection limit, utilizing the data distribution for each location.

In the Bayesian model fitting, we used a built-in censored data function from the brms package (Buerkner, 2017) to address the censored data points. In data plots, we see a large spread in TKN values at all three Snohomish County location (e.g., Fig. 4.54; Fig. 4.59). This intra-location variablity in TKN coincides with low-traffic locations (Fig. 4.60, top left), and may be caused by variability in the timing of plant debris that is swept into storm drains in less urban areas. Additionally, two samples from Tacoma watersheds had abnormally low detection limits while the remaining samples from these watersheds had much higher TKN values, resulting in a wide spread of data points for the two Tacoma watersheds (see Fig. 4.50).

The calculated RSR for the Bayesian TKN model is 0.79, indicating an unsatisfactory model. However, the large spread in TKN values in five of the 14 fitting locations is a clear contributor to the high RSR value. Because the Stormwater Heatmap links chemicals of concern to spatial landscape predictors (rather than temporal ones), we think that the high RSR is not reflective of the model’s utility in this context.

Location-specific intercepts arising from the nested structure of the model’s random effects showed little variability between locations (Fig. 4.62), indicating that we might expect the TKN model to fit well at new locations.

The TKN model is relatively robust to changes in the model structure. Removing influential data points (2 for TKN, one of which was randomly generated for a censored data point) resulted in a moderate impact to the sqrt_traffic coefficient, which changed by 5.8%. Changing the variance structure resulted in a small impact to the coefficient for sqrt_traffic (3.2% change) and little impact to the other coefficients. Removing the random effects nesting structure from the model had a moderate effect on the coefficients (up to 5.4% change).

Model validation was performed using TKN data from 10 outfalls. Nine of these catchments were located in high-traffic roads/ highways, while the remaining catchment was centered on a high-traffic bridge but also included the surrounding high-density residential area. The model prediction intervals overlap the validation data very nicely, indicating good fit of the model to the validation data (Fig. 4.81). These results are promising, but additional data in commercial, industrial and low-density residential areas would be valuable for validating model fit in a variety of environments.

In general, we think the TKN model is a solid improvement over land use category models, and can be relied upon to show locations of high TKN in the Puget Sound Basin. The model also has been shown to provide reliable predictions for TKN magnitude at new (but mostly high-traffic roadway) locations.

## Translating Equations to the Stormwater Heatmap

The process of translating water quality equations for constituents of concern to the Stormwater Heatmap involved manipulation of the spatial predictor data grid followed by application of Bayesian COC equations to calculate estimated COC concentrations by location.

### Preparing Spatial Predictors Images Across the Puget Sound Basin

Multi-band spatial landscape predictor images across the Puget Sound basin (described in Chapter 2: High-Resolution Land Cover) were first transformed (traffic, CO2_road and CO2_transport predictors were square-root transformed, and devAge was squared) then scaled and centered using the mean and standard deviation from the fourteen S8 catchment locations. The resulting images were convolved with a circular kernel of 100 meters to calculate the mean value in the neighborhood of each point. Finally, the convolved spatial predictor images were clamped at +/- 2 standard deviations of the S8 catchment location values. Clamping was necessary to avoid extrapolating beyond the range where linear relationships between spatial predictors and COCs may no longer apply. Figure 4.85 shows the range of predictor values for the S8 catchments, along with the minimum and maximum predictor values across the Puget Sound basin.

Figure 4.85. NEED TO ADD sqrt_CO2_transport AND UPDATE not_greenBE to be the one using the TNC tree cover!!! Plot showing the range of scaled and centered values for each spatial predictor found in the S8 Data (orange bar and black data points), along with the extent of values found within the Puget Sound basin (gray lines). The magenta bars extend the orange bars to the values at which spatial predictors were clamped. Note that the upper extent of the grey line for sqrt_traffic should be at 62, but has been truncated to provide an appropriate scale for the other predictors.

Clamping the traffic predictor was especially important because traffic levels on high-volume roads such as freeways can far exceed the traffic levels in our monitored catchments. The maximum raw traffic value was clamped at 5,000 AADT, whereas on highways in the Puget Sound basin (not analyzed in this study) traffic can reach 100,000 AADT.

Based on highway runoff data from the whole United States (USGS Highway Runoff DataBase, accessed 11/01/21), we observed that linear equations relating TSS to traffic volume provides estimates that are too high when traffic is allowed to reach levels near 240,000. When traffic volumes are clamped at 31,000 ADT, the estimates are more believable. This suggests that, for at least some COCs, traffic may be a linear predictor at the traffic volumes in the S8 catchments, but may reach an asymptote at high traffic volumes. It also suggests that caution should be used when interpreting estimates for areas with exceptionally high traffic volumes.

EVA: look in to this – with the TSS equations for high traffic areas, were we using traffic or sqrt_traffic? It will make a difference in the commentary here…

### Preparing Visualization Layers

To visualize COC concentrations via a webmap, we applied water quality regression equations to the prepared spatial predictor images to generate predicted concentrations for the study area.

Using the scaled and centered multi-band spatial landscape predictor image, we employed the following steps to generate heatmap layers.

Regression equations were applied to the convolved spatial predictor dataset (regression equations were applied using spatial predictor values in each grid box.)

### Applying Mixed Effects Equations to the Stormwater Heatmap

The equations that were fitted to the S8 monitoring data required a specified variance structure to account for heteroskedasticity, as well as a nested structure for the random effects. Homoskedasticity is an important pre-requisite to constructing linear models between predictors and dependent variables, and both the locations and the agencies had some level of variability in the spreads of their data (Fig. 4.68).

Figure 4.68 Spread of the S8 monitoring data for total suspended solids (TSS) by location (top) and by reporting agency (bottom), with color coding by reporting agency. Note the variability in spread per location (e.g., blue boxes vs. green boxes) as well as per reporting agency (e.g., red boxes vs. green boxes).

With respect to translating mixed effects model equations to the Stormwater Heatmap, the equations are substantially simplified. Predicting mean COC concentrations allow the variance structure and nested random effects terms to go to their mean values of 0. Furthermore, rainfall and the summer predictor, where applicable, are also set to their mean value of 0. Thus the generic COC equation:

ln(COCijk)=β0+β1*raini+β2*summeri+β3*predictor1ijk+β4*predictor2ijk+ak+aj|k+ϵijk

ak∼N(0,σagency)

aj|k∼N(0,σlocation)

ϵijk∼N(0,σk)

becomes simplified to:

ln(COC)=β0+β1*0+β2*0+β3*predictor1+β4*predictor2+0+0+0

and finally:

ln(COC)=β0+β3*predictor1+β4*predictor2

## Citations:

Amini, H., M. Danesh-Yazdi, Q. Di, W. Requia, Y. Wei, Y. AbuAwad, L. Shi, M. Franklin, C.-M. Kang, J. M. Wolfson, P. James, R. Habre, Q. Zhu, J. S. Apte, Z. J. Andersen, X. Xing, C. Hultquist, I. Kloog, F. Dominici, P. Koutrakis, and J. Schwartz. 2022. Hyperlocal super-learned PM2.5 components across the contiguous US. Research Square.  https://doi.org/10.21203/rs.3.rs-1745433/v2.

Bookter, A. 2017. Copper and zinc in urban runoff: Phase 1 - Potential pollutant sources and release rates. Washington State Department of Ecology. 17-03-018. https://fortress.wa.gov/ecy/publications/SummaryPages/1703018.html

Buerkner, Paul-Christian, 2017. brms: An R Package for Bayesian Multilevel Models Using Stan. Journal of Statistical Software, 80(1), 1-28. doi:10.18637/jss.v080.i01

Carey, R.O., G.J. Hochmuth, C.J. Martinez, T.H. Boyer, M.D. Dukes, G.S. Toor, J.L. Cisar, 2013. Evaluating nutrient impacts in urban watersheds: Challenges and research opportunities. Environmental Pollution, Vol 173, pp 138-149. https://doi.org/10.1016/j.envpol.2012.10.004

Davis, A.P, M Shokouhian, S. Ni, 2001. Loading estimates of lead, copper, cadmium, and zinc in urban runoff from specific sources. Chemosphere 44 (5), pp 997-1009. https://doi.org/10.1016/S0045-6535(00)00561-0

Faraway, Julian J. Extending the Linear Model with R: Generalized Linear, Mixed Effects and Nonparametric Regression Models. Boca Raton (FL): Chapman & Hall/CRC; 2006.

Gurney, K.R., J. Liang, R. Patarasuk, Y. Song, J. Huang, and G. Roest. 2019. Vulcan: High-Resolution Annual Fossil Fuel CO2 Emissions in USA, 2010-2015, Version 3. ORNL DAAC, Oak Ridge, Tennessee, USA. https://doi.org/10.3334/ORNLDAAC/1741

Helsel, Dennis R. Statistics for Censored Environmental Data Using Minitab and R. 2nd Edition. Hoboken, NJ: John Wiley & Sons, Inc; 2012.

Hobbs, W., B. Lubliner, N. Kale, and E. Newell, 2015. Western Washington NPDES Phase 1 Stormwater Permit: Final Data Characterization 2009-2013. Washington State Department of Ecology, Olympia, WA. Publication No. 15-03-001. Available online at: https://fortress.wa.gov/ecy/publications/SummaryPages/1503001.html

Jacobson, CR, 2011. Identification and quantification of the hydrological impacts of imperviousness in urban catchments: A review. Journal of Environmental Management 96 (6), pp 1438-1448. https://doi.org/10.1016/j.jenvman.2011.01.018

Jani, Jariani, Yun-Ya Yang, Mary G. Lusk, Gurpal S. Toor, 2020. Composition of nitrogen in urban residential stormwater runoff: concentrations, loads, and source characterization of nitrate and organic nitrogen. PLoS One. https://doi.org/10.1371/journal.pone.0229715

Kay, M, 2022. tidybayes: Tidy Data and Geoms for Bayesian Models. R package version 3.0.2. Available online at: http://mjskay.github.io/tidybayes/

Lee, Lopaka, 2020. NADA: Nondetects and Data Analysis for Environmental Data. R package version 1.6-1.1. Available online at: https://CRAN.R-project.org/package=NADA

Loy A, Hofmann H, 2014. HLMdiag: A suite of diagnostics for hierarchical linear models in R. Journal of Statistical Software, 56(5), 1-28. doi:10.18637/jss.v056.i05.

Ma, Yukun, Shihui Wang, Xiaoyue Zhang, Zhenyao Shen, 2021. Transport process and source contribution of nitrogen in stormwater runoff from urban catchments. Environmental Pollution. Vol 289 (15). https://doi.org/10.1016/j.envpol.2021.117824

Moriasi D N, Arnold J G, Van Liew M W, Bingner R L, Harmel R D, Veith T L, 2007. Model evaluation guidelines for systematic quantification of accuracy in watershed simuations. American Society of Agricultural and Biological Engineers. Vol 50(3): 885-900.

Pesaresi M, Ehrlich D, Florczyk A, Freire S, Julea A, Kemper T, Soille P, Syrris V (2015): GHS- BUILT R2015B - GHS built-up grid, derived from Landsat, multitemporal (1975, 1990, 2000, 2014) European Commission, Joint Research Centre (JRC) [Dataset] PID: https://data.jrc.ec.europa.eu/dataset/jrc-ghsl-ghs_built_ldsmt_globe_r2015b

Pinheiro J, Bates D, DebRoy S, Sarkar D, R Core Team, 2021. nlme: Linear and Nonlinear Mixed Effects Models. R package version 3.1-152. Available online at: https://CRAN.R-project.org/package=nlme

Pope, L.M and H.E. Bevans, 1987. Relation of urban landuse and dry-weather, storm, and snowmelt flow characteristics to stream-water quality, Shanganunga Creek Basin. Topeka Kansas, US Geologic Survey Water-Supply. Paper 2283.

R Core Team, 2021. R: A language and environment for statistical computing. R Foundation for Statistical Computing, Vienna, Austria. Available online at: https://www.R-project.org/

Robertson J, Howe E. High-resolution raster dataset of Annual Daily Traffic on roads in the Puget Sound trough during 2016 and 2018. [Manuscript in preparation]

Robertson J, Pierce KB, Nilsen C and Howe E. High-resolution 7-class Land Cover Dataset of the Puget Sound. [Manuscript in preparation]

Sharma, A.K., L. Vezzaro, H. Birch, K. Arnbjerg-Nielsen, P.S. Mikkelsen, 2016. Effect of climate change on stormwater runoff characteristics and treatment efficiencies of stormwater retention ponds: a case study from Denmark using TSS and Cu as indicator pollutants. Springerplus 5(1): 1984. doi: 10.1186/s40064-016-3103-7

Singh J, Knapp H V, and Demissie M, 2004. Hydrologic modeling of the Iroquois River watershed using HSPF and SWAT. ISWS CR 2004-08. Champaign, Ill.: Illinois State Water Survey. Available online at: https://www.isws.illinois.edu/pubdoc/cr/iswscr2004-08.pdf

Stan Development Team, 2022. Stan Modeling Language User’s Guide and Reference Manual. Available online at: https://mc-stan.org/users/documentation/.

Taylor, K.G., and P.N. Owens, 2009. Sediments in urban river basins: a review of sediment-contaminant dynamics in an environmental system conditioned by human activities. Journal of Soils and Sediments 9(4): 281-303. DOI:[10.1007/s11368-009-0103-z](https://doi.org/10.1007/s11368-009-0103-z)

Thornton, M. M., Shrestha, R., Wei, Y., Thornton, P. E., & Kao, S.-C. (2022). Daymet: Daily Surface Weather Data on a 1-km Grid for North America, Version 4 R1 (Version 4.1). ORNL Distributed Active Archive Center. https://doi.org/10.3334/ORNLDAAC/2129

United States Census Bureau: 2010 Census Blocks. Website visited 2/8/2024. Available online at: https://www.census.gov/geographies/mapping-files/time-series/geo/tiger-line-file.2010.html#list-tab-790442341

U.S. Environmental Protection Agency, 2009, Statistical analysis of groundwater monitoring data at RCRA facilities, Unified guidance, EPA 530-R-09-007.

USGS Highway Runoff DataBase. Website visited 11/01/2021. Available online at: https://www.usgs.gov/software/hrdb-highway-runoff-database-software-page

USGS National Elevation Dataset. Website visited 2/8/2024. Available online at: https://pubs.usgs.gov/of/2011/1127/ned.html

Waschbusch, R.J., W.R. Selbig, and R.T. Bannerman, 1999. Sources of Phosphorus in Stormwater and Street Dirt from Two Urban Residential Basins In Madison, Wisconsin, 1994–95. U.S. Geological Survey Water-Resources Investigations Report 99–4021.

Washington Department of Commerce: Puget Sound Mapping Project. Website visited 2/8/2024. Available online at: https://www.commerce.wa.gov/serving-communities/growth-management/puget-sound-mapping-project

Yang, YY., and Lusk, M.G., 2018. Nutrients in Urban Stormwater Runoff: Current State of the Science and Potential Mitigation Options. Curr Pollution Rep 4, 112–127. https://doi.org/10.1007/s40726-018-0087-7

Zhou, B., J. Radosavljevic, C.T. Parsons, F. Rezanezhad, B. MacVicar, K. Pi, Y. Deng, A.R. Shahvaran, S. Slowinski, B. Gharabaghi, P. Van Cappellen, 2026. Phosphorus in urban stormwater runoff: a review of sources, transport pathways, and control solutions. Environmental Reviews 34, pp. 1-24, 10.1139/er-2025-0247

Zhu, Ling, Yukun Ma, Ashantha Goonetilleke, 2004. Fingerprinting to trace sources of suspended soilds in the transport of heavy metals in urban stormwater runoff. Environmental Pollution, Vol 363(1). https://doi.org/10.1016/j.envpol.2024.125088

Zuur, AF, Ieno EN, Walker NJ, Saveliev AA, Smith GM. Mixed Effects Models and Extensions in Ecology with R. New York (NY): Springer; 2009.

Phase I Permittees include: cities of Tacoma and Seattle; King, Snohomish, Pierce and Clark counties; Ports of Tacoma and Seattle.  For this study, all data were used with the exception of those from Clark County (outside of the Puget Sound region) and Port of Seattle (too different from all other Puget Sound watersheds).

As described in Faraway (2006), pg 156, REML estimation should not be used to compare models with different fixed effects because REML estimates the random effects by considering linear combinations of the data that remove the fixed effects. If the fixed effects change, the likelihoods of the two models will not be directly comparable. However, REML is generally considered to give better estimates for the random effects, so final models were fitted with REML